| E/ECE/324 E/ECE/TRANS/505 |  | Add.3/Rev.2/Amend.2 |  |  |
| --- | --- | --- | --- | --- |
| 7 August 2008 |  |  |  |  |

# AGREEMENT

CONCERNING THE ADOPTION OF UNIFORM TECHNICAL PRESCRIPTIONS
FOR WHEELED VEHICLES, EQUIPMENT AND PARTS WHICH CAN BE FITTED
AND/OR BE USED ON WHEELED VEHICLES AND THE CONDITIONS FOR RECIPROCAL RECOGNITION OF APPROVALS GRANTED ON THE BASIS OF THESE PRESCRIPTIONS /

(Revision 2, including the amendments that entered into force on 16 October 1995)

_________

Addendum 3:  Regulation No. 4

## Revision 2 - Amendment 2

#### Supplement 13 to the original version of the Regulation - Date of entry into force:  11 July 2008

Uniform provisions concerning the approval of devices for the illumination of rear registration plates of power-driven vehicles and their trailers

_________

### UNITED NATIONS

Insert a new paragraph 1.4., to read:

"1.4.	References made in this Regulation to standard (étalon) filament lamp(s) and to Regulation No. 37 shall refer to Regulation No. 37 and its series of amendments in force at the time of application for type approval."

Paragraph 2., amend to read:

"2.	APPLICATION FOR APPROVAL

….

(b)	a brief technical description stating in particular, with the exception of lamps with non-replaceable light sources:

(i)	the category or categories of filament lamp prescribed; this filament lamp category shall be one of those contained in Regulation No. 37 and its series of amendments in force at the time of application for type approval; and/or

…. "

Insert new paragraphs 5.6. to 5.6.3., to read:

"5.6.	In the case of replaceable filament lamp(s):

5.6.1.	Any category or categories of filament lamp(s) approved according to Regulation No. 37 may be used, provided that no restriction on the use is made in Regulation No. 37 and its series of amendments in force at the time of application for type approval.

5.6.2.	The design of the device shall be such that the filament lamp can be fixed in no other position but the correct one.

5.6.3.	The filament lamp holder shall conform to the characteristics given in IEC Publication 60061.  The holder data sheet relevant to the category of filament lamp used, applies."

-----

/	Former title of the Agreement:

Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.

GE.08-