| E/ECE/324/Rev.1/Add.22/Rev.3/Amend.3−E/ECE/TRANS/505/Rev.1/Add.22/Rev.3/Amend.3 |  |  |
| --- | --- | --- |
|  |  | 7 July 2011 |

Agreement

Concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions*

(Revision 2, including the amendments which entered into force on 16 October 1995)

Addendum 22: Regulation No. 23

Revision 3 – Amendment 3

Supplement 17 to the original version of the Regulation - Date of entry into force: 23 June 2011

Uniform provisions concerning the approval of reversing lights for power-driven vehicles and their trailers

UNITED NATIONS

Table of Contents, amend to read:

"…

Annexes

1	Communication concerning the approval or extension or refusal or withdrawal of approval or production definitely discontinued of a type of reversing lamp pursuant to Regulation No. 23...................................................

2	Examples of arrangements of approval marks....................................................

3	Photometric measurements.................................................................................

4	Minimum requirements for conformity of production control procedures.........

5	Minimum requirements for sampling by an inspector......................................"

Text of the Regulation,

Paragraph 8., amend to read:

"8.		Colour of light emitted

The colour of the light emitted inside the field of the light distribution grid defined at paragraph 2. of Annex 3 shall be white. To check these colorimetric characteristics, the test procedure described in paragraph 7. of this Regulation shall be applied.  Outside this field no sharp variation of colour shall be observed.

However, for lamps equipped with non-replaceable light sources (filament lamps and other), the colorimetric characteristics should be verified with the light sources present in the lamp, in accordance with relevant subparagraphs of paragraph 7.1. of this Regulation."

Paragraph 9.2., amend to read:

"9.2.	The minimum requirements for conformity of production control procedures set forth in Annex 4 to this Regulation shall be complied with."

Paragraph. 9.3. amend to read:

"9.3.	The minimum requirements for sampling by an inspector set forth in Annex 5 to this Regulation shall be complied with."

Annex 4, shall be deleted.

Annex 5 (former), renumber as Annex 4.

Annex 4 (new),

Paragraph 2.5., amend to read:

"2.5.	Criteria governing acceptability

The manufacturer is responsible for carrying out a statistical study of the test results and for defining, in agreement with the competent authority, criteria governing the acceptability of his products in order to meet the specifications laid down for verification of conformity of products in paragraph 9.1. of this Regulation.

The criteria governing the acceptability shall be such that, with a confidence level of 95 per cent, the minimum probability of passing a spot check in accordance with Annex 5 (first sampling) would be 0.95."

Annex 6 (former), renumber as Annex 5.

*	*	Former title of the Agreement: Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.