|  | E/ECE/324/Rev.1/Add.36/Rev.7/Amend.3−E/ECE/TRANS/505/Rev.1/Add.36/Rev.7/Amend.3 |  |
| --- | --- | --- |
|  |  | 6 August 2013 |

Agreement

Concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions*

(Revision 2, including the amendments which entered into force on 16 October 1995)

_________

Addendum 36 – Regulation No. 37

Revision 7 - Amendment 3

Supplement 40 to the 03 series of amendments – Date of entry into force: 15 July 2013

Uniform provisions concerning the approval of filament lamps for use in approved lamp units of power-driven vehicles and of their trailers

_________

UNITED NATIONS

Paragraph 3.6.3., amend to read:

"3.6.3.	The colour of the light emitted shall be measured by the method specified in Annex 5.  Each measured value shall lie within the required tolerance area1.  Moreover, in the case of filament lamps emitting white light, the measured values shall not deviate more than 0.020 unit in the x and/or y direction from a point of choice on the Planckian locus (IEC Publication 15.2 Colorimetry, 1986).  Filament lamps for use in light signalling devices shall meet the requirements as specified in paragraph 2.4.2. of IEC Publication 60809, Amendment 5 to Edition 2.

______________

1 	For Conformity of Production purposes of amber and red colour only, at least 80 per cent of the measuring results shall lie within the required tolerance area."

Paragraphs 8.3. and 8.4., amend to read:

"8.3.	As from the period after the entry into force of the Supplements to the 03 series of amendments, as indicated for each category in the table for group 3 in Annex 1, no filament lamps of these categories or of the types within these categories shall be used in lamps submitted for type approval.

8.4.	However, for the period after entry into force of the Supplements to the 03 series of amendments, as indicated in the table for group 3 in Annex 1, Contracting Parties applying this Regulation may continue to grant approvals for lamps in which filament lamps of these categories or of the types within these categories are used, provided that these lamps are intended as replacement parts for installation on vehicles in use."

Annex 1, the list of categories of filament lamps and their sheets, amend to read:

"....

Group 1

Without general restrictions:

| Category |  | Sheet number(s) |
| --- | --- | --- |
| H1 | *6 | H1/1 to 3 |
| H3 | *6 | H3/1 to 4 |
| H4 |  | H4/1 to 5 |
| H7 |  | H7/1 to 4 |
| H8 |  | H8/1 to 4 |
| H8B |  | H8/1 to 4 |
| H9 | *3 | H9/1 to 4 |
| H9B | *3 | H9/1 to 4 |
| H10 |  | H10/1 to 3 |
| H11 |  | H11/1 to 4 |
| H11B |  | H11/1 to 4 |
| H13 |  | H13/1 to 4 |
| H15 |  | H15/1 to 5 |
| H16 |  | H16/1 to 4 |
| H16B |  | H16/1 to 4 |
| H17 |  | H17/1 to 6 |
| H21W | *2 | H21W/1 to 2 |
| H27W/1 |  | H27W/1 to 3 |
| H27W/2 |  | H27W/1 to 3 |
| HB3 |  | HB3/1 to 4 |
| HB4 |  | HB4/1 to 4 |
| HIR2 |  | HIR2/1 to 3 |
| HS1 | *6 | HS1/1 to 5 |
| HS2 | *6 | HS2/1 to 3 |
| HS5 |  | HS5/1 to 4 |
| - HS5A | *5 | HS5A/1 to 3 |
| PSX24W | *2 | P24W/1 to 3 |
| PSX26W | *2 | PSX26W1 to 3 |
| PX24W | *2 | P24W/1 to 3 |
| S2 | *6 | S1/S2/1 to 2 |

Group 2

Only for use in signalling lamps, cornering lamps, reversing lamps and rear registration plate lamps:

| Category |  | Sheet number(s) |  |
| --- | --- | --- | --- |
| C5W | *6 | C5W/1 |  |
| H6W |  | H6W/1 |  |
| H10W/1 |  | H10W/1 to 2 |  |
| HY6W |  | H6W/1 |  |
| HY10W |  | H10W/1 to 2 |  |
| HY21W |  | H21W/1 to 2 |  |
| P13W |  | P13W/1 to 3 |  |
| P21W | *6 | P21W/1 to 2 |  |
| P21/4W |  | P21/4W/1 | (P21/5W/2 to 3) |
| P21/5W | *6 | P21/5W/1 to 3 |  |
| P24W |  | P24W/1 to 3 |  |
| P27W |  | P27W/1 to 2 |  |
| P27/7W |  | P27/7W/1 to 3 |  |
| PR21W |  | PR21W/1 | (P21W/2) |
| PR21/5W |  | PR21/5W/1 | (P21/5W/2 to 3) |
| PS19W |  | P19W/1 to 3 |  |
| PS24W |  | P24W/1 to 3 |  |
| PSY19W |  | P19W/1 to 3 |  |
| PSY24W |  | P24W/1 to 3 |  |
| PW13W |  | P13W/1 to 3 |  |
| PW16W |  | PC16W/1 to 3 |  |
| PWR16W |  | PC16W/1 to 3 |  |
| PWY16W |  | PC16W/1 to 3 |  |
| PW19W |  | P19W/1 to 3 |  |
| PWR19W |  | P19W/1 to 3 |  |
| PWY19W |  | P19W/1 to 3 |  |
| PW24W |  | P24W/1 to 3 |  |
| PWR24W |  | P24W/1 to 3 |  |
| PWY24W |  | P24W/1 to 3 |  |
| PY21W |  | PY21W/1 | (P21W/2) |
| PY24W |  | P24W/1 to 3 |  |
| PY27/7W |  | PY27/7W/1 | (P27/7W/2 to 3) |
| R5W | *6 | R5W/1 |  |
| R10W | *6 | R10W/1 |  |
| RR5W |  | R5W/1 |  |
| RR10W |  | R10W/1 |  |
| RY10W | *6 | R10W/1 |  |
| T4W | *6 | T4W/1 |  |
| W2.3W |  | W2.3W/1 |  |
| W3W | *6 | W3W/1 |  |
| W5W | *6 | W5W/1 |  |
| W10W | *6 | W10W/1 |  |
| W15/5W |  | W15/5W/1 to 3 |  |
| W16W |  | W16W/1 |  |
| W21W |  | W21W/1 to 2 |  |
| W21/5W |  | W21/5W/1 to 3 |  |
| WP21W |  | WP21W/1 to 2 |  |
| WPY21W |  | WP21W/1 to 2 |  |
| WR5W |  | W5W/1 |  |
| WR21/5W |  | WR21/5W/1 | (W21/5W/2 to 3) |
| WY5W | *6 | W5W/1 |  |
| WY10W | *6 | W10W/1 |  |
| WY16W |  | W16W/1 |  |
| WY21W |  | WY21W/1 to 2 |  |

Group 3

For replacement purposes only (see transitional provisions of paragraphs 8.3. and 8.4.):

|  |  |  | As specified by transitional provisions in paragraph 8.3. |  | As specified by transitional provisions in paragraph 8.4. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Category |  | Sheet number(s) | Supplement | Period | Supplement | Period |
| C5W | *7, *8 | C5W/1 | 38 | 12 months | 38 | unlimited |
| C21W | *8 | C21W/1 to 2 | 28 | 12 months | 28 | unlimited |
| H1 | *7 | H1/1 to 3 | 38 | 12 months | 38 | unlimited |
| H3 | *7 | H3/1 to 4 | 38 | 12 months | 38 | unlimited |
| H12 |  | H12/1 to 3 | 40 | 24 months | 40 | unlimited |
| H13A |  | H13/1 to 4 | 40 | 24 months | 40 | unlimited |
| H14 |  | H14/1 to 4 | 38 | 12 months | 38 | unlimited |
| HB3A |  | HB3/1 to 4 | 40 | 60 months | 40 | unlimited |
| HB4A |  | HB4/1 to 4 | 40 | 60 months | 40 | unlimited |
| HIR1 | *3 | HIR1/1 to 3 | 40 | 24 months | 40 | unlimited |
| HS1 | *7 | HS1/1 to 5 | 38 | 12 months | 38 | unlimited |
| HS2 | *7 | HS2/1 to 3 | 38 | 12 months | 38 | unlimited |
| - HS6 | *4 | HS6/1 to 4 | 40 | 60 months | 40 | unlimited |
| P19W | *8 | P19W/1 to 3 | 37 | 60 months | 37 | unlimited |
| P21W | *7, *8 | P21W/1 to 2 | 38 | 12 months | 38 | unlimited |
| P21/5W | *7, *8 | P21/5W/1 to 3 | 38 | 12 months | 38 | unlimited |
| PC16W | *8 | PC16W/1 to 3 | 37 | 60 months | 37 | unlimited |
| PCR16W | *8 | PC16W/1 to 3 | 37 | 12 months | 37 | unlimited |
| PCY16W | *8 | PC16W/1 to 3 | 37 | 60 months | 37 | unlimited |
| PR19W | *8 | P19W/1 to 3 | 37 | 12 months | 37 | unlimited |
| PR21/4W | *8 | PR21/4W/1; (P21/5W/2 to 3) | 40 | 24 months | 40 | unlimited |
| PR24W | *8 | P24W/1 to 3 | 37 | 12 months | 37 | unlimited |
| PR27/7W | *8 | PR27/7W/1; (P27/7W/2 to 3) | 40 | 24 months | 40 | unlimited |
| PSR19W | *8 | P19W/1 to 3 | 37 | 12 months | 37 | unlimited |
| PSR24W | *8 | P24W/1 to 3 | 37 | 12 months | 37 | unlimited |
| PY19W | *8 | P19W/1 to 3 | 37 | 60 months | 37 | unlimited |
| R2 |  | R2/1 to 3 | 28 | 12 months | 28 | unlimited |
| R5W | *7, *8 | R5W/1 | 38 | 12 months | 38 | unlimited |
| R10W | *7, *8 | R10W/1 | 38 | 12 months | 38 | unlimited |
| RY10W | *7, *8 | R10W/1 | 38 | 12 months | 38 | unlimited |
| S1 |  | S1/S2/1 to 2 | 28 | 12 months | 28 | unlimited |
| S2 | *7 | S1/S2/1 to 2 | 38 | 12 months | 38 | unlimited |
| S3 |  | S3/1 | 38 | 12 months | 38 | unlimited |
| T1.4W | *8 | T1.4W/1 | 40 | 24 months | 40 | unlimited |
| T4W | *7, *8 | T4W/1 | 38 | 12 months | 38 | unlimited |
| W3W | *7, *8 | W3W/1 | 38 | 12 months | 38 | unlimited |
| W5W | *7, *8 | W5W/1 | 38 | 12 months | 38 | unlimited |
| W10W | *7, *8 | W10W/1 | 38 | 12 months | 38 | unlimited |
| WY2.3W |  | WY2.3W/1 | 40 | 24 months | 40 | unlimited |
| WY5W | *7 | W5W/1 | 40 | 12 months | 40 | unlimited |
| WY10W | *7, *8 | W10W/1 | 38 | 12 months | 38 | unlimited |

*	Tables, Electrical and Photometric characteristics:

Voltage is expressed in V;

Wattage is expressed in W;

Luminous flux is expressed in lm.

In case a category of filament lamp has more than one value of reference luminous flux specified, the value of approximately 12 V for approval of a lighting device and 13.5 V for approval of a light-signalling device shall be applied unless otherwise specified by the regulation used for the approval of the device.

*2	Not for use in passing beam headlamps.

*3	Not for use in front fog lamps marked "B" as defined in Regulation No. 19.

*4	Not for use in Regulation No. 112 headlamps.

*5	Not for use in headlamps other than Regulation No. 113 Class C headlamps.

*6	All types except from 6 V type.

*7	6 V types only.

*8	Only for use in signalling lamps, cornering lamps, reversing lamps and rear registration plate lamps.

Annex 1,

Sheet H17/2, the table, amend to read:

"....

| p | 28.95 | 28.95 |
| --- | --- | --- |
| α | max. 40° | max. 40° |
| Cap PU43t-4 in accordance with IEC Publication 60061 (sheet 7004-171-1) |  |  |
| Electrical and photometric characteristics |  |  |

...."

*	*	Former title of the Agreement: Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.