|  |  |  | 27 November 2013 |
| --- | --- | --- | --- |

Agreement

Concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions*

(Revision 2, including the amendments which entered into force on 16 October 1995)

Addendum 6: Regulation No. 7

Revision 6 – Amendment 2

Supplement 22 to the 02 series of amendments - Date of entry into force: 3 November 2013

Uniform provisions concerning the approval of front and rear position lamps, stop-lamps and end-outline marker lamps for motor vehicles (except motor cycles) and their trailers

UNITED NATIONS

Paragraphs 6.2.4. and 6.2.4.1., amend to read:

"6.2.4.		Moreover,

6.2.4.1.	Throughout the fields defined in the diagrams in Annex 1, the luminous intensity of the light emitted must be not less than 0.05 cd for front and rear position lamps and end-outline marker lamps, not less than 0.3 cd for devices of categories S1, S3 and for those of categories S2 and S4 by day; it shall not be less than 0.07 cd for devices of categories S2 and S4 by night;"

Annex 1, amend to read:

"…

In all cases, the minimum vertical angles of light distribution in space are 15° above and 15° below the horizontal for all categories of devices included in this Regulation, except:

(a)	For lamps intended to be installed with their H plane at a mounting height less than 750 mm above the ground, for which they are 15° above and 5° below the horizontal;

(b)	Optional lamps intended to be installed with their H plane at a mounting height more than 2,100 mm above the ground, for which they are 5° above and 15° below the horizontal;

(c)	For category S3 or S4 stop lamp for which they are 10° above and 5° below	the horizontal.

x

Front position lamps

Vehicle

Driving

direction

45°

80°

Reference axis

"

H plane: "horizontal plane going through the reference centre of the lamp"

20°

80°

Vehicle

Driving

direction

Reference axis

Under the H plane for front position lamps intended to be installed with this plane at a mounting height less than 750 mm above ground.

Vehicle

Vehicle

Front end outline marker lamp (AM)

Reference axis

45°

80°

Driving

direction

Rear end outline marker lamp (RM1, RM2)

Rear position lamps

Reference axis

20°

80°

Driving

direction

Under the H plane for rear position lamps intended to be installed with this plane at a mounting height less than 750 mm above ground.

Reference axis

45°

45°

Driving

direction

Stop-lamps (S1 and S2)

Driving
direction

Under the H plane for stop lamps (S1 and S2) intended to be installed with this plane at a mounting height less than 750 mm above ground.

20°

45°

Reference axis

Stop-lamps (S3 and S4)

"

Annex 2,

Item 9, amend to read:

"9. 	Concise description:

9.1. 	By category of lamp:

For mounting either outside or inside or both:2

Colour of light emitted: red / white2

Number, category and kind of light source(s):

Voltage and wattage:

Light source module specific identification code: .

Only for limited mounting height of equal to or less than 750 mm above the ground: Yes/No2

Geometrical conditions of installation and relating variations, if any: 	."

______________

*	*	Former title of the Agreement: Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.