|  | E/ECE/324/Add.6/Rev.6/Amend.3−E/ECE/TRANS/505/Add.6/Rev.6/Amend.3 |  |
| --- | --- | --- |
|  |  | 17 October 2014 |

Agreement

Concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions*

(Revision 2, including the amendments which entered into force on 16 October 1995)

_________

Addendum 6 – Regulation No. 7

Revision 6 - Amendment 3

Supplement 23 to the 02 series of amendments – Date of entry into force: 9 October 2014

Uniform provisions concerning the approval of front and rear position lamps, stop-lamps and end-outline marker lamps for motor vehicles (except motor cycles) and their trailers

_________

UNITED NATIONS

Paragraph 6.1., amend to read:

"6.1.	The light emitted by each of the two devices supplied shall be in the reference axis, of not less than the minimum intensity and of not more than the maximum intensity specified below:

|  | Minimum luminous intensity in cd | Maximum luminous intensity in cd when used as |  |
| --- | --- | --- | --- |
|  |  | Single lamp | Lamp (single) marked "D" paragraph 4.2.2.6. |
| 6.1.1. Front position lamps, front end-outline marker lamp A or AM | 4 | 140 | 70 |
| 6.1.2. Front position lamps incorporated in a headlamp or front fog lamp | 4 | 140 | - |
| 6.1.3. Rear position lamps, rear end-outline marker lamp 6.1.3.1. R, R1 or RM1 (steady) | 4 | 17 | 8.5 |
| 6.1.3.2. R2 or RM2 (variable) | 4 | 42 | 21 |
| 6.1.4. Stop-lamps |  |  |  |
| 6.1.4.1. S1 (steady) | 60 | 260 | 130 |
| 6.1.4.2. S2 (variable) | 60 | 730 | 365 |
| 6.1.4.3. S3 (steady) | 25 | 110 | 55 |
| 6.1.4.4. S4 (variable) | 25 | 160 | 80 |

"

Paragraph 6.2.4.2., amend to read:

"6.2.4.2.	If a rear position lamp and/or a rear end-outline marker lamp is reciprocally incorporated with a stop-lamp producing either steady or variable luminous intensity, the ratio between the luminous intensities actually measured of the two lamps when turned on simultaneously at the intensity of the rear position lamp or end-outline marker lamp when turned on alone should be at least 5: 1 in the field delimited by the straight horizontal lines passing through ±5° V and the straight vertical lines passing through ±10° H of the light distribution table.

If the one or both of the two reciprocally incorporated lamps contain(s) more than one light source and is (are) considered as a single lamp, the values to be considered are those obtained with all light sources in operation;"

*	*	Former title of the Agreement: Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.