|  |  |  | 14 February 2013 |
| --- | --- | --- | --- |

Agreement

Concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be fitted and/or be used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions*

(Revision 2, including the amendments which entered into force on 16 October 1995)

Addendum 12-H: Regulation No. 13-H

Revision 2 – Amendment 3

Supplement 14 to the original version of the Regulation - Date of entry into force: 
27 January 2013

Uniform provisions concerning the approval of passenger cars with regard to braking

UNITED NATIONS

Paragraph 5.2.20.1., amend to read:

"5.2.20.1. 	With the parking brake released, the service braking system shall be able to fulfil the following requirements:

(a)	With the propulsion system on/off control in the ''On'' (''Run'') position, generate a static total braking force at least equivalent to that required by the Type-0 test for service braking performance as prescribed in paragraph 2.1. of Annex 3 to this Regulation,

(b)	During the first 60 seconds after the propulsion system on/off control has been deactivated to the ''Off'' or ''Lock'' position and/or the ignition key has been removed, three brake applications shall generate a static total braking force at least equivalent to that required by the Type-0 test for service braking performance as prescribed in paragraph 2.1. of Annex 3 to this Regulation, and

(c)	After the period mentioned above, or as from the fourth brake application within the 60 second period, whichever occurs first, generate a static total braking force at least equivalent to that required by the Type-0 test for secondary braking performance as prescribed in paragraph 2.2. of Annex 3 to this Regulation.

It should be understood that sufficient energy is available in the energy transmission of the service braking system."

*	*	Former title of the Agreement: Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.