| E/ECE/324/Add.2/Rev.3/Amend.3−E/ECE/TRANS/505/Add.2/Rev.3/Amend.3 |  |  |
| --- | --- | --- |
|  |  | 7 July 2011 |

Agreement

Concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions*

(Revision 2, including the amendments which entered into force on 16 October 1995)

Addendum 2: Regulation No. 3

Revision 3 – Amendment 3

Supplement 12 to the 02 series of amendments - Date of entry into force: 23 June 2011

Uniform provisions concerning the approval of retro-reflecting devices for power-driven vehicles and their trailers

UNITED NATIONS

Annex 4,

Paragraph 3.1., amend to read:

"3.1.	A colour which satisfies the conditions laid down in Annex 6. This shall be ..."

Annex 6,

Paragraph 2., amend to read:

"2.	When the retro-reflecting device is illuminated by CIE standard illuminant A, with an angle of divergence of 1/3 degrees and an illumination angle of V = H = 0 degrees, or, if this produces a colourless surface reflection, an angle V = +/- 5 degrees, H = 0 degrees, the trichromatic coordinates of the reflected luminous flux must be within the limits according to paragraph 2.30. of Regulation No. 48."

Insert a new paragraph 3., to read:

"3.	Clear retro-reflecting devices must not produce a selective reflection, that is to say, the trichromatic coordinates "x" and "y" of the standard illuminant "A" used to illuminate the retro-reflecting device must not undergo a change of more than 0.01 after reflection by the retro-reflecting device."

Annex 14,

Paragraph 4.1., amend to read:

"4.1.	A colour which satisfies the conditions laid down in Annex 6. This shall be ..."

*	*	Former title of the Agreement: Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.