| E/ECE/324/Rev.1/Add.12/Rev.7/Amend.3−E/ECE/TRANS/505/Rev.1/Add.12/Rev.7/Amend.3 |  |  |
| --- | --- | --- |
|  |  | 23 May 2012 |

Agreement

Concerning the adoption of uniform technical prescriptions for wheeled vehicles, equipment and parts which can be fitted and/or be used on wheeled vehicles and the conditions for reciprocal recognition of approvals granted on the basis of these prescriptions*

(Revision 2, including the amendments which entered into force on 16 October 1995)

Addendum 12: Regulation No. 13

Revision 7 – Amendment 3

Supplement 8 to the 11 series of amendments to the Regulation - Date of entry into force:  
13 April 2012

Uniform provisions concerning the approval of vehicles of categories M, N and O with regard to braking

UNITED NATIONS

Paragraph 5.1.4.7., amend to read:

"5.1.4.7.	It shall be possible to verify, in a frequent and simple way, the correct … available."

Insert a new paragraph 5.1.4.7.1., to read:

"5.1.4.7.1.	Where the operational status is indicated to the driver by warning signals, as specified in this Regulation, it shall be possible at a periodic technical inspection to confirm the correct operational status by visual observation of the warning signals following a power-on."

Paragraph 5.1.4.7.1. (former), renumber as paragraph 5.1.4.7.2.

Paragraph 5.2.1.32., amend to read:

"5.2.1.32.	Subject to the provisions of paragraph 12.4. of this Regulation, all vehicles of the following categories shall be equipped with a vehicle stability function:

(a)	M2, M3, N2 12;

(b)	N3 12 having no more than 3 axles;

(c)	N3 12 with 4 axles, with a maximum mass not exceeding 25 t and a maximum wheel diameter code not exceeding 19.5.

The vehicle stability function shall include roll-over control and directional control and meet the technical requirements of Annex 21 to this Regulation."

Annex 21,

Paragraph 2.1.4., amend to read:

"2.1.4.	Interventions of the vehicle stability function … for this purpose.

Additionally, interventions by systems related to the vehicle stability function (including traction control, trailer stability assist, corner brake control, and other similar functions that use throttle and/or individual torque control to operate and share common components with vehicle stability function) may also be indicated to the driver by this flashing optical warning signal.

Interventions … leaving the driver's seat."

*	*	Former title of the Agreement: Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.