| E/ECE/324 E/ECE/TRANS/505 |  | Rev.1/Add.25/Amend.5 |  |  |
| --- | --- | --- | --- | --- |
| 6 August 2007 |  |  |  |  |

# AGREEMENT

CONCERNING THE ADOPTION OF UNIFORM TECHNICAL PRESCRIPTIONS
FOR WHEELED VEHICLES, EQUIPMENT AND PARTS WHICH CAN BE FITTED
AND/OR BE USED ON WHEELED VEHICLES AND THE CONDITIONS FOR RECIPROCAL RECOGNITION OF APPROVALS GRANTED ON THE BASIS OF THESE PRESCRIPTIONS /

(Revision 2, including the amendments that entered into force on 16 October 1995)

_________

Addendum 25:  Regulation No. 26

## Amendment 5

#### Supplement 1 to the 03 series of amendments - Date of entry into force:  11 June 2007

Uniform provisions concerning the approval OF vehicles with regard to their external projections

_________

### UNITED NATIONS

Paragraph 6.8.1., amend to read:

"6.8.1.	Sheet-metal edges, such as gutter edges and the rails of sliding doors, shall not be permitted unless they are folded back or are fitted with a shield meeting the requirements of this Regulation which are applicable to it.

An unprotected edge shall be considered to be folded back either if it is folded back by approximately 180 º, or if it is folded towards the bodywork in such a manner that it cannot be contacted by a sphere having a diameter of 100 mm.

The requirements of paragraph 5.4. shall not apply to the following sheet metal edges:  rear edge of bonnet and front edge of rear luggage boot."

-----

/	Former title of the Agreement:

Agreement Concerning the Adoption of Uniform Conditions of Approval and Reciprocal Recognition of Approval for Motor Vehicle Equipment and Parts, done at Geneva on 20 March 1958.

GE.07-