|  | United Nations | ECE/TRANS/WP.29/GRE/2023/27 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 11 August 2023 Original: English |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Eighty-ninth session

Geneva, 24-27 October 2023

Item 8 of the provisional agenda

UN Regulation No. 10 (Electromagnetic Compatibility)

Proposal for a new 07 series of amendments to UN Regulation No. 10 (Electromagnetic compatibility)

Submitted by the expert from by the Informal Working Group on Electromagnetic Compatibility*

The text reproduced below was prepared by the experts from the Informal Working Group on Electromagnetic Compatibility (IWG EMC), with the aim to develop a new 07 series of amendments to UN Regulation No.10. The proposed modifications to the current text of the UN Regulation are marked in bold for new or strikethrough for deleted characters.

I.	Proposal

Table of Contents, amend to read:

"Contents

Page**

Regulation

1.	Scope		5

2.	Definitions		5

3.	Application for approval		9

4.	Approval		11

5.	Markings		12

6.	Specification in configurations other than REESS charging mode coupled 
to the power grid		13

7.	Additional specifications in the configuration "REESS charging mode coupled 
to the power grid"		18

8.	Amendment or extension of a vehicle type approval following electrical/electronic
sub-assembly  (ESA) addition or substitution		29

9.	Conformity of production		30

10.	Penalties for non-conformity of production		31

11.	Production definitively discontinued		31

12.	Modification and extension of type approval of a   vehicle or ESA		31

13.	Transitional provisions		32

14.	Names and addresses of Technical Services conducting approval tests and
of Type Approval Authorities		33

Appendix 1 - List of standards referred to in this Regulation		34

Appendix 2 - Vehicle broadband reference limits - Antenna-vehicle separation: 10 m 		36

Appendix 3 - Vehicle broadband reference limits - Antenna-vehicle separation: 3 m		37

Appendix 4 - Vehicle narrowband reference limits - Antenna-vehicle separation: 10 m 		38

Appendix 5 - Vehicle narrowband reference limits - Antenna-vehicle separation: 3 m 		39

Appendix 6 - Electrical/electronic sub-assembly - Broadband reference limits		40

Appendix 7 - Electrical/electronic sub-assembly - Narrowband reference limits		41

Appendix 8 - HV artificial network		42

Annexes

1.	Examples of approval marks		50

2A.	Information document for type approval of a vehicle with respect to 
electromagnetic compatibility		51

2B.	Information document for type approval of an electric/electronic sub-assembly 
with respect to electromagnetic compatibility		55

3A.	Communication concerning the approval or extension or refusal or withdrawal of approval or 
production definitively discontinued of a type of vehicle/component/separate technical unit 
with regard to Regulation No. 10 		57

3B.	Communication concerning the approval or extension or refusal or withdrawal of approval or 
	production definitively discontinued of a type of electrical/electronic sub-assembly with regard 
to Regulation No. 10		59

4.	Method of measurement of radiated broadband electromagnetic emissions from vehicles		61

Appendix 1 		67

5.	Method of measurement of radiated narrowband electromagnetic emissions from vehicles		74

Appendix 1		77

6.	Method of testing for immunity of vehicles to electromagnetic radiation		78

Appendix 1		86

7.	Method of measurement of radiated broadband electromagnetic emissions from 
electrical/electronic sub-assemblies (ESAs)		92

Appendix 1		96

8.	Method of measurement of radiated narrowband electromagnetic emissions from 
electrical/electronic sub-assemblies		98

9.	Method(s) of testing for immunity of electrical/electronic sub-assemblies to 
electromagnetic radiation		100

Appendix 1		106

Appendix 21 - Typical TEM cell dimensions		108

Appendix 32 - Absorber chamber test		109

Appendix 43 - BCI test		111

10.	Method(s) of testing for immunity to and emission of transients of electrical/electronic 
sub-assemblies		113

11.	Method(s) of testing for emission of harmonics generated on AC power lines from vehicle		114

Appendix 1		116

12.	Method(s) of testing for emission of voltage changes, voltage fluctuations and flicker 
on AC power lines from vehicle		118

Appendix 1		120

13.	Method(s) of testing for emission of radiofrequency conducted disturbances on AC or 
DC power lines from vehicles		122

Appendix 1		124

14.	RESERVED Method(s) of testing for emission of radiofrequency conducted disturbances on 
network and telecommunication access from vehicles 		126

Appendix 1		129

15.	Method of testing for immunity of vehicles to Electrical Fast Transient/Burst 
disturbances conducted along AC and DC power lines		133

Appendix 1		135

16.	Method of testing for immunity of vehicles to surges conducted along AC and DC 
power lines		137

Appendix 1 - Vehicle in configuration "REESS charging mode coupled to the power grid"		139

17.	Method(s) of testing for emission of harmonics generated on AC power lines from 
an ESA		141

Appendix 1		143

18.	Method(s) of testing for emission of voltage changes, voltage fluctuations and flicker 
on AC power lines from an ESA		144

Appendix 1		146

19.	Method(s) of testing for emission of radiofrequency conducted disturbances on AC 
or DC power lines from an ESA		147

Appendix 1		149

20.	RESERVED Method(s) of testing for emission of radiofrequency conducted disturbances on 
network and telecommunication access from an ESA		150

Appendix 1		152

21.	Method of testing for immunity of an ESA to Electrical Fast Transient/Burst 
disturbances conducted along AC and DC power lines		153

Appendix 1		155

22.	Method of testing for immunity of ESAs to surges conducted along AC and DC 
power lines		156

Appendix 1 - ESA in configuration "REESS charging mode coupled to the power grid"		158"

Paragraph 1.3., amend to read:

"1.3.	It covers:

(a)	Requirements regarding the immunity to radiated and conducted disturbances for functions related to direct control of the vehicle, related to driver, passenger and other road users' protection, related to disturbances, which would cause confusion to the driver or other road users, related to vehicle data bus functionality, related to disturbances, which would affect vehicle statutory data;

(b)	Requirements regarding the control of unwanted radiated and conducted emissions to protect the intended use of electrical or electronic equipment at own or adjacent vehicles or nearby, and the control of disturbances from accessories that may be retrofitted to the vehicle;

(c)	Additional requirements for vehicles and ESAs providing coupling systems for charging the REESS regarding the control of emissions and immunity from this connection between vehicle and power grid.

Note: The immunity levels stated in this regulation ensure robustness of vehicle systems in the electromagnetic environment and therefore addresses one of the external environments to be considered for functional safety (e.g. in ISO 26262)."

Paragraph 2.12., amend to read:

"2.12.	"Immunity related functions" are the following functions; this list is not exhaustive and shall be adapted to the technical evolution of vehicle/technology:

(a)	Functions related to the direct control of the vehicle:

(i)	By degradation or change in: e.g. engine, gear, brake, 	suspension, active steering, speed limitation devices;

(ii)	By affecting drivers position: e.g. seat or steering wheel 	positioning;

(iii)	By affecting driver's visibility: e.g. dipped beam, windscreen 	wiper, indirect vision systems, blind spot systems.

(b)	Functions related to driver, passenger and other road user protection:

(i)	E.g. airbag and safety restraint systems, accident emergency calling systems (e-call, AECS, ERA GLONASS, …);

(c)	Functions which, when disturbed, cause confusion to the driver or other road users:

(i)	Optical disturbances: incorrect operation of e.g. direction 	indicators, stop lamps, end outline marker lamps, rear position 	lamp, light bars for emergency system, wrong information 	from warning indicators, lamps or displays related to functions 	in subparagraphs (a) or (b) which might be observed in the 	direct view of the driver;

(ii)	Acoustical disturbances: incorrect operation of e.g. anti-theft 	alarm, horn.

(d)	Functions related to vehicle data bus functionality:

(i)	By blocking data transmission on vehicle data bus-systems, 	which are used to transmit data, required to ensure the correct 	functioning of other immunity related functions.

(e)	Functions which when disturbed affect vehicle statutory data: e.g. tachograph, odometer;

(f)	Function related to charging mode when coupled to the power grid:

(i)	For vehicle test: by leading to unexpected vehicle motion;

(ii)	For ESA test: by leading to an incorrect charging condition (e.g. over-current, over-voltage)."

Paragraph 2.20., amend to read:

"2.20.		"Signal/control port” means port intended for the interconnection of components of an ESA, or between an ESA and local AE (Ancillary Auxiliary Equipment) and used in accordance with relevant functional specifications (for example for the maximum length of cable connected to it). Examples include RS-232, Universal Serial Bus (USB), High-Definition Multimedia Interface (HDMI), IEEE Standard 1394 (“Fire Wire”). For vehicle in charging mode this includes Control Pilot signal, PLC technology used on Control Pilot signal line, CAN."

Insert new paragraphs 2.26. to 2.32., to read:

"2.26	“Automated Driving System (ADS)” means the vehicle hardware and software that are collectively capable of performing the entire Dynamic Driving Task (DDT) on a sustained basis. (1)

NOTE: ADS means the driver, during the driving, is not responsible for Dynamic Driving Task, unless otherwise specified in other UN Regulations.

- 2.27. 	“Dynamic Driving Task (DDT)” means the real-time operational and tactical functions required to operate the vehicle. (1)

2.28.	“Residential environment” refers to clause 3.1.14 of IEC 61000-6-3: 2020.

Area of land designated for domestic dwellings where the mains power within these locations is directly connected to the low-voltage (lower than 1000Va.c. and 1500 Vd.c.) public mains network.

Note 1 to entry: Examples of residential locations are: houses, apartments, farm buildings housing people.

Note 2 to entry: A dwelling can be a single building, separate building or a separate section of a larger building.

Note 3 to entry: Within these locations it is expected to operate a radio receiver within a distance of 10 m from the equipment.

Note 4 to entry: Domestic dwellings are places for one or more people to live.

2.29.	“Non-residential environment” refers to clause 3.1.12 of IEC 61000-6-4: 2018.

Location characterized by a separate power network, supplied from a high- or medium-voltage transformer, dedicated for the supply of the installation

Note 1 to entry: Industrial locations can generally be described by the existence of an installation with one or more of the following characteristics:

• 	items of equipment installed and connected together and working simultaneously;

• 	significant amount of electrical power generated, transmitted and/or consumed;

• 	frequent switching of heavy inductive or capacitive loads;

• 	high currents and associated magnetic fields;

• 	presence of industrial, high power scientific and medical (ISM) equipment (for example, welding machines).

The electromagnetic environment at an industrial location is predominantly produced by the equipment and installation present at the location. There are types of industrial locations where some of the electromagnetic phenomena appear in a more severe degree than in other installations.

Example locations include metalworking, pulp and paper, chemical plants, car production, farm building, high voltage areas of airports.

2.30. 	“Acoustic Vehicle Alerting System (AVAS)” as defined in the latest series of amendments to UN Regulation No. 138.

2.31.	“Accident Emergency Call Systems (AECS)” as defined in the latest series of amendments to UN Regulation No. 144.

2.32.	“Auxiliary Equipment (AE)” means equipment needed to exercise or monitor the operation of the test object."

Insert a new footnote (1), to read:

"(1): 	Those definitions are based on the definitions contained in ECE/TRANS/WP.29/2022/58, Annex 1."

Paragraph 6.1.2., amend to read:

"6.1.2.	Before testing, the Technical Service has to prepare a test plan in conjunction with the manufacturer, which contains at least mode of operation, stimulated function(s), monitored function(s), pass/fail criterion(criteria) and intended emissions.

Depending on the intended operating conditions of the propulsion, the test plan shall define the minimum number of steady state operating conditions of the vehicle, selectable by the driver or by the control system. This(ese) test conditions shall consider multiple propulsion system operating strategies which can be activated permanently by the driver or the vehicle control system and shall be based on documentation provided in Annexes 2A and 2B (e.g. risk analysis)."

Paragraph 6.3.2.4., to be deleted.

Paragraph 6.4.2.1., amend to read:

"6.4.2.1.	If tests are made using the method described in Annex 6, with ISO 11451-2 method, the field strength shall be 30 volts/m rms (root mean squared) in over 90 per cent of the 20 to 2,000 MHz frequency band and a minimum of 25 volts/m rms over the whole 20 to 2,000 MHz frequency band. The field strength shall be 10 volts/m rms (root mean squared) in over 90 per cent of the 2,000 to 6,000 MHz frequency band and a minimum of 8 volts/m rms over the whole 2,000 to 6,000 MHz frequency band.

If tests are made using the method described in Annex 6, with ISO 11451-4 BCI method the current shall be 60 mA rms (root mean squared)."

Paragraph 6.8.2.1., amend to read:

"6.8.2.1.	If tests are made using the methods described in Annex 9, the immunity test levels shall be 60 volts/m root-mean-square (rms) for the 150 mm stripline testing method, 15 volts/m rms for the 800 mm stripline testing method, 75 volts/m rms for the Transverse Electromagnetic Mode (TEM) cell testing method, 60 mA rms for the bulk current injection (BCI) testing method and 30 volts/m rms for the free field testing method in over 90 per cent of the 20 to 2,000 MHz frequency band, and to a minimum of 50 volts/m rms for the 150 mm stripline testing method, 12.5 volts/m rms for the 800 mm stripline testing method, 62.5 volts/m rms, for the TEM cell testing method, 50 mA rms for the bulk current injection (BCI) testing method and 25 volts/m rms for the free field testing method over the whole 20 to 2,000 MHz frequency band.

The immunity to electromagnetic radiation of ESA representative of its type shall be tested by the method(s) as described in Annex 9.:

Test severity in over 90 per cent of the 20 to 6,000 MHz frequency band are given in Table 2a.

Test severity for the minimum test Level over the whole 20 to 6,000 MHz frequency band given in Table 2b.

Table 2a

|  | Test Level in over 90 per cent of the 20 to 6,000 MHz frequency band |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency range | Stripline | TEM cell | BCI | ALSE | Reverberation chamber |
| 20 to 2,000 MHz | 60 V/m | 75 V/m | 60 mA | 30 V/m | 21 V/m |
| 2,000 to 6,000 MHz | Not applicable | Not applicable | Not applicable | 10 V/m | 7 V/m |

Table 2b

|  | Minimum Test Level over the whole 20 to 6,000 MHz frequency band |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency range | Stripline | TEM cell | BCI | ALSE | Reverberation chamber |
| 20 to 2,000 MHz | 50 V/m | 62,5 V/m | 60 mA | 25 V/m | 18 V/m |
| 2,000 to 6,000 MHz | Not applicable | Not applicable | Not applicable | 8 V/m | 6 V/m |

"

Paragraph 6.9.1., amend to read:

"6.9.1.	Method of testing

The immunity of ESA representative of this type shall be tested by the method(s) according to ISO 7637-2 as described in Annex 10, with the test levels given in Table 23.

# Table 23

Immunity of ESA

| Test pulse number | Immunity test level | Functional status for systems: |  |
| --- | --- | --- | --- |
|  |  | Related to immunity related functions | Not related to immunity related functions |
| 1 | III | C | D |
| 2a | III | B | D |
| 2b | III | C | D |
| 3a/3b | III | A | D |
| 4 | III | B (for ESA which shall be operational during engine start phases) C (for other ESA) | D |

"

Paragraph. 6.10.4., to be deleted.

Paragraphs 6.10.5. to 6.10.7., renumber to read:

"6.10.54.		Emission of transient conducted disturbances generated by ESAs on 12/24 V supply lines.

ESAs that are not switched, contain no switches or do not include inductive load need not be tested for transient conducted emission and shall be deemed to comply with paragraph 6.7.

6.10.65.	The loss of function of receivers during the immunity test, when the test signal is within the receiver bandwidth (RF exclusion band) as specified for the specific radio service/product in the harmonized international EMC standard, does not necessarily lead to fail criteria.

6.10.76.	RF transmitters shall be tested in the transmit mode. Wanted emissions (e.g. from RF transmitting systems) within the necessary bandwidth and out of band emissions are disregarded for the purpose of this Regulation. Spurious emissions are subject to this Regulation.

6.10.76.1.	"Necessary bandwidth": For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions (Article 1, No. 1.152 of the International Telecommunication Union (ITU) Radio Regulations).

6.10.76.2.	"Out-of-band Emissions": Emission on a frequency or frequencies immediately outside the necessary bandwidth which results from the modulation process, but excluding spurious emissions (Article 1, No. 1.144 of the ITU Radio Regulations).

6.10.76.3.	"Spurious emission": In every modulation process additional undesired signals exist. They are summarized under the expression "spurious emissions". Spurious emissions are emissions on a frequency or frequencies, which are outside the necessary bandwidth and the level of which may be reduced without affecting the corresponding transmission of information. Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products, but exclude out-of-band emissions (Article 1 No. 1.145 of the ITU Radio Regulations)."

Insert a new paragraph 6.10.7., to read:

"6.10.7.	Trolleybuses: AC / DC mains portion of the vehicle propulsion system shall be excluded from this Regulation."

Paragraph 7.1.2., amend to read:

"7.1.2.		Before testing the Technical Service has to prepare a test plan in conjunction with the manufacturer, for the configuration "REESS charging mode coupled to the power grid" configuration which contains at least mode of operation, stimulated function(s), monitored function(s), pass/fail criterion (criteria) and intended emissions.

Depending on the available charging modes of the REESS, the test plan shall define the minimum number of test conditions of the vehicle, selectable by the driver or by the control system and shall be based on documentation provided in Annexes 2A and 2B (e.g. risk analysis). Refer to flow charts in Annexes where REESS charge mode is applicable."

Paragraph 7.1.3., amend to read:

"7.1.3.		A vehicle in configuration "REESS charging mode coupled to the power grid" should be tested with the charging harness delivered by the manufacturer in line with flow charts provided in Annexes 4, 6, 11, 12, 13, 15 and 16. In this case, the cable shall be type approved as part of the vehicle."

Paragraph 7.1.4., amend to read:

"7.1.4.		Artificial networks

AC Power mains shall be applied to the vehicle / ESA through 50 µH/50  AMN(s) as defined in Appendix 8 clause 4.

DC Power mains shall be applied to the vehicle / ESA through 5 µH/50  DC-charging-AN(s) as defined in Appendix 8 clause 3.

High voltage power line shall be applied to the ESA through a 5 µH/50  HV-AN(s) as defined in Appendix 8 clause 2.

Signal port lines, control port lines or wired network port lines should be applied to the vehicle / ESA through an AAN as defined in Appendix 8 clause 5."

Paragraph 7.3.2.1., amend to read:

"7.3.2.1.		If measurements are made using the method described in Annex 11, the limits for input current ≤ 16 A per phase are those defined in IEC 61000-3-2 and given in Table 34.

# Table 34

Maximum allowed harmonics (input current ≤ 16 A per phase)

| Harmonic number n | Maximum authorized harmonic current A |
| --- | --- |
| Odd harmonics |  |
| 3 | 2.3 |
| 5 | 1.14 |
| 7 | 0.77 |
| 9 | 0.40 |
| 11 | 0.33 |
| 13 | 0.21 |
| 15 ≤ n ≤ 39 | 0.15x15/n |
| Even harmonics |  |
| 2 | 1.08 |
| 4 | 0.43 |
| 6 | 0.30 |
| 8 ≤ n ≤ 40 | 0.23x8/n |

"

Paragraph 7.3.2.2., amend to read:

"7.3.2.2.		If measurements are made using the method described in Annex 11, the limits for input current > 16 A and ≤ 75 A per phase are those defined in IEC 61000-3-12, and given in given in Table 45, Table 56 and Table 67.

# Table 45

# Maximum allowed harmonics (input current > 16 A and ≤ 75 A per phase) for single phase or other than balanced three-phase equipment

| Minimum Rsce | Acceptable individual harmonic current In/I1 % |  |  |  |  |  | Maximum current harmonic ratio % |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I3 | I5 | I7 | I9 | I11 | I13 | THD | PWHD |
| 33 | 21.6 | 10.7 | 7.2 | 3.8 | 3.1 | 2 | 23 | 23 |
| 66 | 24 | 13 | 8 | 5 | 4 | 3 | 26 | 26 |
| 120 | 27 | 15 | 10 | 6 | 5 | 4 | 30 | 30 |
| 250 | 35 | 20 | 13 | 9 | 8 | 6 | 40 | 40 |
| ≥ 350 | 41 | 24 | 15 | 12 | 10 | 8 | 47 | 47 |
| Relative values of even harmonics lower or equal to 12 shall be lower than 16/n %. Even harmonics greater than 12 are taken into account in the Total Harmonic Distorsion (THD) and Partial Weighted Harmonic Distorsion (PWHD) the same way than odd harmonics. Linear interpolation between successive values of Short Circuit Ratio of an Equipment (Rsce) is authorized. |  |  |  |  |  |  |  |  |

# Table 56

# Maximum allowed harmonics (input current > 16 A and ≤ 75 A per phase) for balanced three-phase equipment

| Minimum Rsce | Acceptable individual harmonic current In/I1 % |  |  |  | Maximum current harmonic ratio % |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | I5 | I7 | I11 | I13 | THD | PWHD |
| 33 | 10.7 | 7.2 | 3.1 | 2 | 13 | 22 |
| 66 | 14 | 9 | 5 | 3 | 16 | 25 |
| 120 | 19 | 12 | 7 | 4 | 22 | 28 |
| 250 | 31 | 20 | 12 | 7 | 37 | 38 |
| ≥ 350 | 40 | 25 | 15 | 10 | 48 | 46 |
| Relative values of even harmonics lower or equal to 12 shall be lower than 16/n %. Even harmonics greater than 12 are taken into account in the THD and PWHD the same way than odd harmonics. Linear interpolation between successive values of Rsce is authorized. |  |  |  |  |  |  |

# Table 67

# Maximum allowed harmonics (input current > 16 A and ≤ 75 A per phase) for balanced three-phase equipment under specific conditions

| Minimum Rsce | Acceptable individual harmonic current In/I1 % |  |  |  | Maximum current harmonic ratio % |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | I5 | I7 | I11 | I13 | THD | PWHD |
| 33 | 10.7 | 7.2 | 3.1 | 2 | 13 | 22 |
| ≥ 120 | 40 | 25 | 15 | 10 | 48 | 46 |
| Relative values of even harmonics lower or equal to 12 shall be lower than 16/n %. Even harmonics greater than 12 are taken into account in the THD and PWHD the same way than odd harmonics." |  |  |  |  |  |  |

Paragraphs 7.5.2. to 7.5.2.2., amend to read:

"7.5.2.	Vehicle type approval limit for vehicle charged in residential environment

7.5.2.1.	If measurements are made using the method described in Annex 13, the limits on AC power lines are those defined in IEC 61000-6-3 and given in Table 78.

# Table 78

# Maximum allowed radiofrequency conducted disturbances on AC power lines

| Frequency (MHz) | Limits and detector |
| --- | --- |
| 0.15 to 0.5 | 66 to 56 dBµV (quasi-peak) 56 to 46 dBµV (average) (linearly decreasing with logarithm of frequency) |
| 0.5 to 5 | 56 dBµV (quasi-peak) 46 dBµV (average) |
| 5 to 30 | 60 dBµV (quasi-peak) 50 dBµV (average) |

7.5.2.2.	If measurements are made using the method described in Annex 13, the limits on DC power lines are those defined in IEC 61000-6-3
and given in Table 89.

# Table 89

# Maximum allowed radiofrequency conducted disturbances on DC power lines

| Frequency (MHz) | Limits and detector |
| --- | --- |
| 0.15 to 0.5 | 79 dBµV (quasi-peak) 66 dBµV (average) |
| 0.5 to 30 | 73 dBµV (quasi-peak) 60 dBµV (average) |

"

Insert new paragraphs 7.5.3. to 7.5.3.4., to read:

"[7.5.3	Vehicle type approval limit for vehicle charged only in other environment than residential environment.

7.5.3.1.	For specific vehicles which are charged only with charging stations located in area characterized by a separate power network, supplied from a high or medium voltage transformer, dedicated for the supply of the installation (buses, heavy duty trucks, etc..), it can be appropriate to apply limits from IEC 61000-6-4.

7.5.3.2.	In this case, the manufacturer shall provide a statement that the vehicle can be used in "REESS charging mode coupled to the power grid" only in area characterized by a separate power network, supplied from a high or medium voltage transformer, dedicated for the supply of the installation. The manufacturer shall provide a statement that the vehicle shall be charged in non-residential environment only. This information shall be made publicly available following the type approval.

7.5.3.3	If measurements are made using the method described in Annex 13, the limits on AC power lines are those defined in IEC 61000-6-4 and given in Table 10.

# Table 10

# Maximum allowed radiofrequency conducted disturbances on AC power lines

| Frequency (MHz) | Limits and detector |
| --- | --- |
| 0.15 to 0.5 | 79 dBµV (quasi-peak) 66 dBµV (average) |
| 0.5 to 30 | 73 dBµV (quasi-peak) 60 dBµV (average) |

7.5.3.4.	If measurements are made using the method described in Annex 13, the limits on DC power lines are those defined in IEC 61000-6-4
and given in Table 11.

# Table 11

# Maximum allowed radiofrequency conducted disturbances on DC power lines

| Frequency (MHz) | Limits and detector |
| --- | --- |
| 0.15 to 0.5 | 89 dBµV (quasi-peak) 76 dBµV (average) |
| 0.5 to 30 | 83 dBµV (quasi-peak) 70 dBµV (average) |

]"

Paragraph 7.6., amend to read:

"7.6.		Specifications concerning emission of radiofrequency conducted disturbances on wired network port from vehicles.

No longer applicable."

Paragraphs 7.6.1. to 7.6.2.1., to be deleted.

Paragraph 7.7.2.1., amend to read:

"7.7.2.1.	If tests are made using the method described in Annex 6, the field strength shall be 30 volts/m rms (root mean squared) in over 90 per cent of the 20 to 2,000 MHz frequency band and a minimum of 25 volts/m rms over the whole 20 to 2,000 MHz frequency band. The field strength shall be 10 volts/m rms (root mean squared) in over 90 per cent of the 2,000 to 6,000 MHz frequency band and a minimum of 8 volts/m rms over the whole 2,000 to 6,000 MHz frequency band."

Paragraph 7.8.2.2., amend to read:

"7.8.2.2.	The vehicle representative of its type shall be considered as complying with immunity requirements if, during the tests performed in accordance with Annex 15, there shall be no degradation of performance of "immunity related functions", according to paragraph 2.2. of Annex 6 paragraph 2.1.2. of Annex 15."

Paragraph 7.9.2.1., amend to read:

"7.9.2.1.		If tests are made using the methods described in Annex 16, the immunity test levels shall be:

(a)	For AC power lines: ±2 kV test voltage in open circuit between line and earth and ±1 kV between lines (pulse 1.2 µs / 50 µs), with a rise time (Tr) of 1.2 µs, and a hold time (Th) of 50 µs. Each surge shall be applied five times with a maximum delay of 1 minute between each pulse. This has to be applied for the following phases: 0, 90, 180 and 270°,

(b)	For DC power lines: ±0.5 kV test voltage in open circuit between line and earth and ±0.5 kV between lines (pulse 1.2 µs / 50 µs) with a rise time (Tr) of 1.2 µs, and a hold time (Th) of 50 µs. Each surge shall be applied five times with a maximum delay of 1 minute."

Paragraph 7.9.2.2., amend to read:

"7.9.2.2.	The vehicle representative of its type shall be considered as complying with immunity requirements if, during the tests performed in accordance with Annex 16, there shall be no degradation of performance of "immunity related functions", according to paragraph 2.2. of Annex 6 paragraph 2.1.2. of Annex 16."

Paragraph 7.11.2.1., amend to read:

"7.11.2.1.		If measurements are made using the method described in Annex 17, the limits for input current ≤ 16 A per phase are those defined in IEC 61000-3-2 and given in Table 120.

# Table 120

# Maximum allowed harmonics (input current ≤ 16 A per phase)

| Harmonic number n | Maximum authorized harmonic current A |
| --- | --- |
| Odd harmonics |  |
| 3 | 2.3 |
| 5 | 1.14 |
| 7 | 0.77 |
| 9 | 0.40 |
| 11 | 0.33 |
| 13 | 0.21 |
| 15 ≤ n ≤ 39 | 0.15x15/n |
| Even harmonics |  |
| 2 | 1.08 |
| 4 | 0.43 |
| 6 | 0.30 |
| 8 ≤ n ≤ 40 | 0.23x8/n |

"

Paragraph 7.11.2.2., amend to read:

"7.11.2.2.		If measurements are made using the method described in Annex 17, the limits for input current > 16 A and ≤ 75 A per phase are those defined in IEC 61000-3-12 and given in Table 131, Table 142 and Table 153.

# Table 131

# Maximum allowed harmonics (input current > 16 A and ≤ 75 A per phase) for single phase or other than balanced three-phase equipment

| Minimum Rsce | Acceptable individual harmonic current In/I1 % |  |  |  |  |  | Maximum current harmonic ratio % |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I3 | I5 | I7 | I9 | I11 | I13 | THD | PWHD |
| 33 | 21.6 | 10.7 | 7.2 | 3.8 | 3.1 | 2 | 23 | 23 |
| 66 | 24 | 13 | 8 | 5 | 4 | 3 | 26 | 26 |
| 120 | 27 | 15 | 10 | 6 | 5 | 4 | 30 | 30 |
| 250 | 35 | 20 | 13 | 9 | 8 | 6 | 40 | 40 |
| ≥ 350 | 41 | 24 | 15 | 12 | 10 | 8 | 47 | 47 |
| Relative values of even harmonics lower or equal to 12 shall be lower than 16/n %. Even harmonics greater than 12 are taken into account in the THD and PWHD in the same way than odd harmonics. Linear interpolation between successive values of Rsce is authorized. |  |  |  |  |  |  |  |  |

# Table 142

# Maximum allowed harmonics (input current > 16 A and ≤ 75 A per phase) for balanced three-phase equipment

| Minimum Rsce | Acceptable individual harmonic current In/I1 % |  |  |  | Maximum current harmonic ratio % |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | I5 | I7 | I11 | I13 | THD | PWHD |
| 33 | 10.7 | 7.2 | 3.1 | 2 | 13 | 22 |
| 66 | 14 | 9 | 5 | 3 | 16 | 25 |
| 120 | 19 | 12 | 7 | 4 | 22 | 28 |
| 250 | 31 | 20 | 12 | 7 | 37 | 38 |
| ≥ 350 | 40 | 25 | 15 | 10 | 48 | 46 |
| Relative values of even harmonics lower or equal to 12 shall be lower than 16/n %. Even harmonics greater than 12 are taken into account in the THD and PWHD in the same way as odd harmonics. Linear interpolation between successive values of Rsce is authorized. |  |  |  |  |  |  |

# Table 153

# Maximum allowed harmonics (input current > 16 A and ≤ 75 A per phase) for balanced three-phase equipment under specific conditions

| Minimum Rsce | Acceptable individual harmonic current In/I1 % |  |  |  | Maximum current harmonic ratio % |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | I5 | I7 | I11 | I13 | THD | PWHD |
| 33 | 10.7 | 7.2 | 3.1 | 2 | 13 | 22 |
| ≥ 120 | 40 | 25 | 15 | 10 | 48 | 46 |
| Relative values of even harmonics lower or equal to 12 shall be lower than 16/n %. Even harmonics greater than 12 are taken into account in the THD and PWHD in the same way as odd harmonics." |  |  |  |  |  |  |

Paragraph 7.13.2.1., amend to read:

"7.13.2.1.	If measurements are made using the method described in Annex 19, the limits on AC power lines are those defined in IEC 61000-6-3 and given in Table 164.

# Table 164

Maximum allowed radiofrequency conducted disturbances on AC power lines

| Frequency (MHz) | Limits and detector |
| --- | --- |
| 0.15 to 0.5 | 66 to 56 dBµV (quasi-peak) 56 to 46 dBµV (average) (linearly decreasing with logarithm of frequency) |
| 0.5 to 5 | 56 dBµV (quasi-peak) 46 dBµV (average) |
| 5 to 30 | 60 dBµV (quasi-peak) 50 dBµV (average) |

"

Paragraph 7.13.2.2., amend to read:

"7.13.2.2.		If measurements are made using the method described in Annex 19, the limits on DC power lines are those defined in IEC 61000-6-3 and given in Table 175.

# Table 175

Maximum allowed radiofrequency conducted disturbances on DC power lines

| Frequency (MHz) | Limits and detector |
| --- | --- |
| 0.15 to 0.5 | 79 dBµV (quasi-peak) 66 dBµV (average) |
| 0.5 to 30 | 73 dBµV (quasi-peak) 60 dBµV (average) |

"

Paragraph 7.14., amend to read:

"7.14.		Specifications concerning emission of radiofrequency conducted disturbances wired network port from ESA

No longer applicable."

Paragraphs 7.14.1. to 7.14.2.1., to be deleted.

Paragraph 7.15.2.2., amend to read:

"7.15.2.2.	The ESA representative of its type shall be considered as complying with immunity requirements if, during the tests performed in accordance with Annex 21, there shall be no degradation of performance of "immunity related functions", according to paragraph 2.2. of Annex 9 the charging function as defined in paragraph 2.1. of Annex 21."

Paragraph 7.16.2.1., amend to read:

"7.16.2.1.	If tests are made using the methods described in Annex 22, the immunity test levels shall be:

(a)	For AC power lines: ±2 kV test voltage in open circuit between line and earth and ±1 kV between lines (pulse 1.2 µs / 50 µs), with a rise time (Tr) of 1.2 µs, and a hold time (Th) of 50 µs. Each surge shall be applied five times with a maximum delay of 1 minute between each pulse. This has to be applied for the following phases: 0, 90, 180 and 270°,

(b)	For DC power lines: ±0.5 kV test voltage in open circuit between line and earth and ±0.5 kV between lines (pulse 1.2 µs / 50 µs) with a rise time (Tr) of 1.2 µs, and a hold time (Th) of 50 µs. Each surge shall be applied five times with a maximum delay of 1 minute."

Paragraph 7.16.2.2., amend to read:

"7.16.2.2.		The ESA representative of its type shall be considered as complying with immunity requirements if, during the tests performed in accordance with Annex 22, there shall be no degradation of performance of "immunity related functions", according to paragraph 2.2. of Annex 9 the charging function as defined in paragraph 2.1. of Annex 22."

Paragraph 7.17.1., amend to read:

"7.17.1.		Method of testing

The emission of ESA representative of its type shall be tested by the method(s) according to ISO 7637-2, as described in Annex 10 for the levels given in Table 187.

# Table 187

# Maximum allowed pulse amplitude

|  | Maximum allowed pulse amplitude for |  |
| --- | --- | --- |
| Polarity of pulse amplitude | Vehicles with 12 V systems | Vehicles with 24 V systems |
| Positive | +75 V | +150 V |
| Negative | –100 V | –450 V |

"

Paragraph 7.18.2.1., amend to read:

"7.18.2.1.	If tests are made using the methods described in Annex 9, the immunity test levels shall be 60 volts/m rms for the 150 mm stripline testing method, 15 volts/m rms for the 800 mm stripline testing method, 75 volts/m rms for the Transverse Electromagnetic Mode (TEM) cell testing method, 60 mA rms for the Bulk Current Injection (BCI) testing method and 30 volts/m rms for the free field testing method in over 90 per cent of the 20 to 2,000 MHz frequency band, and to a minimum of 50 volts/m rms for the 150 mm stripline testing method, 12.5 volts/m rms for the 800 mm stripline testing method, 62.5 volts/m rms, for the TEM cell testing method, 50 mA rms for the bulk current injection (BCI) testing method and 25 volts/m rms for the free field testing method over the whole 20 to 2,000 MHz frequency band.

The immunity to electromagnetic radiation of ESA representative of its type shall be tested by the method(s) as described in Annex 9 :

- 	for the levels in over 90 per cent of the 20 to 6,000 MHz frequency band given in Table 19a.

- 	for the minimum test Level over the whole 20 to 6,000 MHz frequency band given in Table 19b.

Table 19a

|  | Test Level in over 90 per cent of the 20 to 6,000 MHz frequency band |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency range | Stripline | TEM cell | BCI | ALSE | Reverberation chamber |
| 20 to 2,000 MHz | 60 V/m | 75 V/m | 60 mA | 30 V/m | 21 V/m |
| 2,000 to 6,000 MHz | Not applicable | Not applicable | Not applicable | 10 V/m | 7 V/m |

Table 19b

|  | Minimum Test Level over the whole 20 to 6,000 MHz frequency band |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency range | Stripline | TEM cell | BCI | ALSE | Reverberation chamber |
| 20 to 2,000 MHz | 50 V/m | 62,5 V/m | 50 mA | 25 V/m | 18 V/m |
| 2,000 to 6,000 MHz | Not applicable | Not applicable | Not applicable | 8 V/m | 6 V/m |

"

Paragraph 7.18.2.2., amend to read:

"7.18.2.2.		The ESA representative of its type shall be considered as complying with immunity requirements if, during the tests performed in accordance with Annex 9, there shall be no degradation of performance of "immunity related functions"the charging function as defined in paragraph 2.3. of Annex 9."

Paragraph 7.19.1., amend to read:

"7.19.1		Method of testing

The immunity of ESA representative of its type shall be tested by the method(s) according to ISO 7637-2, as described in Annex 10 with the test levels given in Table 1820.

# Table 1820

# Immunity of ESA

| Test pulse number | Immunity test level | Functional status for systems: |  |
| --- | --- | --- | --- |
|  |  | Related to immunity related functions | Not related to immunity related functions |
| 1 | III | C | D |
| 2a | III | B | D |
| 2b | III | C | D |
| 3a/3b | III | A | D |

"

Paragraphs 7.20.1. to 7.20.3., to be deleted.

Paragraphs 7.20.4. and 7.20.5., renumber to read:

"7.20.41.		Vehicles and / or ESA which are intended to be used in "REESS charging mode coupled to the power grid" in the configuration connected to a
DC-charging station with a length of a DC network cable (cable between the DC charging station and the vehicle plug) shorter than 30 m do not have to fulfil the requirements of paragraphs 7.5., 7.8., 7.9., 7.13., 7.15., 7.16.

In this case, the manufacturer shall provide a statement that the vehicle and/or ESA can be used in "REESS charging mode coupled to the power grid" only with cables shorter than 30 m. This information shall be made publicly available following the type approval.

7.20.52.		Vehicles and / or ESA which are intended to be used in "REESS charging mode coupled to the power grid" in the configuration connected to a
local / private DC-charging station without additional participants do not have to fulfil requirements of paragraphs 7.5., 7.8., 7.9., 7.13., 7.15., 7.16.

In this case, the manufacturer shall provide a statement that the vehicle
and / or ESA can be used in "REESS charging mode coupled to the power grid" only with a local/private DC charging station without additional participants. This information shall be made publicly available following the type approval. "

Insert new paragraphs 13.3. to 13.3.5., to read:

"[13.3.	Transitional provisions applicable to the 07 series of amendments

13.3.1.  	As from the official date of entry into force of the 07 series of amendments, no Contracting Party applying this UN Regulation shall refuse to grant or refuse to accept UN type-approvals under this UN Regulation as amended by the 07 series of amendments.

13.3.2. 	As from 1 September 2029, Contracting Parties applying this UN Regulation shall not be obliged to accept UN type-approvals to the preceding series of amendments, first issued after 1 September 2029.

13.3.3.  	Contracting Parties applying this Regulation shall continue to accept type approvals issued according to any of the preceding series of amendments to this Regulation first issued before 1 September 2029.

13.3.4. 	Contracting Parties applying this Regulation may grant type approvals according to any preceding series of amendments to this Regulation.

13.3.5.  	Contracting Parties applying this Regulation shall continue to grant extensions of existing approvals to any preceding series of amendments to this Regulation.]"

Appendix 1,

Paragraph 5., amend to read:

"5.	ISO-EN 17025 "General requirements for the competence of testing and calibration laboratories", secondthird edition 20172005 and Corrigendum: 2006."

Paragraph 7., amend to read:

"7.	ISO 11452 "Road vehicles - Electrical disturbances by narrowband radiated electromagnetic energy - Component test methods":

Part 1:	General and definitions (ISO 11452-1, third edition 2005 and

Amd1: 2008);

Part 2:	Absorber-lined chamber (ISO 11452-2, second edition 2004);

Part 3:	Transverse electromagnetic mode (TEM) cell (ISO 11452-3,

third edition 2016);

Part 4:	Bulk current injection (BCI) (ISO 11452-4, fourth edition 20112022);

Part 5:	Stripline (ISO 11452-5, second edition 2002).

Part 11: Reverberation chamber (ISO 11452-11, first edition 2010)."

Insert a new paragraph 21., to read:

"21. 	CISPR 16-1-1 “Specification for radio disturbance and immunity measuring apparatus and methods - Part 1-1: Radio disturbance and immunity measuring apparatus - Measuring apparatus ", edition 4.0 2015-09."

Appendix 8,

Paragraph 1., figure 1, amend to read:

# "Figure 1

# Example of 5 H AN schematic

| Legend |  |
| --- | --- |
| L1: 5 µH | A: Port to power supply |
| C1: 0,1 µF | P: Port to Vehicle or ESA |
| C2: 1 µF (default value) | B: Ground |
| R1: 1 kΩ | MEP: Measuring Port" |

“

Paragraph 2., figures 3, 4 and 5, amend to read:

# "Figure 3

# Example of 5 H / HV-AN schematic

| Legend |  |
| --- | --- |
| L1: 5 µH | HV supply: High Voltage power supply |
| C1: 0,1 µF | Vehicle / ESA HV: High Voltage of Vehicle or ESA |
| C2: 0,1 µF (default value) | MEP: Measuring Port |
| R1: 1 kΩ | GND: Ground |
| R2: 1 MΩ (discharging C2 to > 50 Vdc within 60 s) |  |

If unshielded HV ANs are used in a single shielded box, then there shall be an inner shield between the HV ANs as described in Figure 4."

# "Figure 4

# Example of 5 H HV AN combination in a si ngle shielded box

| Legend |  |
| --- | --- |
| L1: 5 µH | HV supply: High Voltage power supply (positive and negative) |
| C1: 0,1 µF | Vehicle / ESA HV: High Voltage of Vehicle or ESA (positive and negative) |
| C2: 0,1 µF (default value) | MEP: Measuring Port |
| R1: 1 kΩ | GND: Ground |
| R2: 1 MΩ (discharging C2 to > 50 Vdc within 60 s) |  |

An optional impedance matching network may be used to simulate common mode / differential mode impedance seen by the ESA plugged on HV power supply (see Figure 5). "

# "Figure 5 –

# Impedance matching network attached between HV ANs and ESA

| Legend |  |
| --- | --- |
| L1: 5 µH | HV supply: High Voltage power supply (positive and negative) |
| C1: 0,1 µF | Vehicle / ESA HV: High Voltage of Vehicle or ESA (positive and negative) |
| C2: 0,1 µF (default value) | MEP: Measuring Port |
| R1: 1 kΩ | GND: Ground |
| R2: 1 MΩ (discharging C2 to > 50 Vdc within 60 s) | ZDI-CM: Differential and common-mode impedance" |

Appendix 8,

Paragraph 3., amend to read:

"3.		Direct Current charging Artificial Networks (DC-charging-AN)

For a vehicle/ESA in charging mode connected to a DC power supply, a 5 µH / 50 Ω DC-charging-AN as defined in Figure 6 shall be used.

Measurement ports of DC-charging-AN(s) shall be terminated with 50  loads.

The DC-charging-AN impedance ZPB (tolerance 20 %) in the measurement frequency range of 0,1 MHz to 100 MHz is shown in Figure 7. It is measured between the terminals “Vehicle/ESA HV” and “GND” (of Figure 6) with a 50  load on the measurement port and with terminals “HV Supply” and “GND” (of Figure 6) short circuited.

…"

Paragraph 4., amend to read:

"4.		Artificial Mains networks (AMN)

For a vehicle/ESA in charging mode connected to an AC power mains, a 50 µH / 50 Ω-AMN as defined in CISPR 16-1-2 clause 4.4 shall be used.

Measurement ports of AMN(s) shall be terminated with 50  loads. "

Paragraph 5., amend to read:

"5.		Asymmetric artificial network (AAN)

Currently, different technologies for signal/control port lines and/or wired network port lines are used for the communication between charging station and vehicle/ESA. Therefore, a distinction between some specific signal/control port lines and/or wired network port lines (for example, control pilot line, CAN lines) is necessary.

Measurement ports of AAN(s) shall be terminated with 50  loads.

AANs that are defined in 5.1., 5.2., 5.3. and 5.4. are used for unshielded signal/control port lines and/or wired network port lines.

If shielded signal/control port lines are used, then shielded AANs defined in CISPR 32:2015 Annex G, Figures G.10 and G.11 should be used."

Paragraph 5.1., amend to read:

"5.1.		Signal/Control port with symmetric lines

An asymmetric artificial network (AAN) to be connected between the vehicle and the charging station or any associated auxiliary equipment (AE) used to simulate communication is defined in CISPR 16-1-2 Annex E clause E.2 (T network circuit) (see example in Figure 8).

…"

Paragraph 5.2., amend to read:

"5.2.		Wired network port with PLC on power lines

If an original charging station can be used for the test, an AAN and/or AMN/DC-charging-AN might not be required for PLC communication.

If the presence of the AMN/DC-charging-AN prevents proper PLC communication with the original charging station or if the PLC communication needs to be simulated by means of a piece of associated auxiliary equipment (e.g. a PLC modem) instead of the original charging station, it is necessary to add an AAN between the AE (e.g. the PLC modem) and the AMN/DC-charging-AN output (vehicle side), as shown in Figure 9.

…"

Annex 1, amend to read:

"Examples of approval marks

Model A

(See paragraph 5.2. of this Regulation)

10 R – 076 2439

a = 6 mm min

The above approval mark affixed to a vehicle or ESA shows that the vehicle type concerned has, with regard to electromagnetic compatibility, been approved in the Netherlands (E 4) pursuant to Regulation No. 10 under approval No. 075 2439. The approval number indicates that the approval was granted according to the requirements of Regulation No. 10 as amended by the 076 series of amendments.

Model B

(See paragraph 5.2. of this Regulation)

|  | 10 06 2439. 33 00 1628. |  |
| --- | --- | --- |

a = 6 mm min

The above approval mark affixed to a vehicle or ESA shows that the vehicle type concerned has, with regard to electromagnetic compatibility, been approved in the Netherlands (E 4) pursuant to Regulations Nos. 10 and 33. The approval numbers indicate that, at the date when the respective approvals were given, Regulation No. 10 included the 06 series of amendments and Regulation No. 33 was still in its original form."

Annex 2A,

Item 10, amend to read:

"10a. 	Internal combustion engineList propulsion systems, auxiliary engines and components of the charging system: ……………………………………...	"

Insert a new item 10b, to read:

"10b. 	List modes of operations (selectable by the driver or the system) which are selected during the type approval test:……………………………………. "

Item 64, amend to read:

"64.	Vehicle equipped with 24 GHz short-range radar equipment: yes/no/optional.1

The applicant for type approval shall also supply, where appropriate:

Appendix 1 	A list with make(s) and type(s) of all electrical and/or electronic components 
		concerned by this Regulation (see paragraphs 2.9. and 2.10. of this Regulation) 
		and not previously listed.

Appendix 2:	Schematics or drawing of the general arrangement of electrical and/or 
		electronic components (concerned by this Regulation) and the general wiring 
		harness arrangement.

Appendix 3: Description of vehicle chosen to represent the type:

Body style:

Left or right hand drive:

Wheelbase:

Appendix 4: Relevant test report(s) supplied by the manufacturer from a test laboratory 
		accredited to ISO 17025 and recognized by the Type Approval Authority for 
		the purpose of drawing up the type approval certificate."

Item 66, amend to read:

"66.	List every available charging mode (Charging current: direct current / alternating current, (number of phases/frequency): 1	 "

Item 67, amend to read:

"67.	Maximum nominalcharging current (in each mode if necessary):

"

Item 68, amend to read:

"68. 	Nominal charging voltage in each mode:

"

Add a new item 73 to read:

"73.	Environment of charging (residential, non-residential)

The applicant for type approval shall also supply, where appropriate:

Appendix 1 	A list with make(s) and type(s) of all electrical and/or electronic components concerned by this Regulation (see paragraphs 2.9. and 2.10. of this Regulation) and not previously listed.

Appendix 2:	Schematics or drawing of the general arrangement of electrical and/or 
	electronic components (concerned by this Regulation) and the general wiring harness arrangement.

Appendix 3: Description of vehicle chosen to represent the type:

Body style:

Left or right hand drive:

Wheelbase:

Appendix 4: Relevant test report(s) supplied by the manufacturer from a test laboratory accredited to ISO 17025 and recognized by the Type Approval Authority for the purpose of drawing up the type approval certificate."

Annex 3A, amend to read:

"Annex 3A

Communication

(Maximum format: A4 (210 x 297 mm))

issued by:		Name of administration:

......................................

......................................

......................................

Concerning: 	Approval granted

Approval extended

Approval refused

Approval withdrawn

Production definitively discontinued

of a type of vehicle/component/separate technical unit2 with regard to Regulation No. 10.

Approval No.: …………………...........		Extension No.: ...........................................

1.	Make (trade name of manufacturer):

2.	Type:

3.	Means of identification of type, if marked on the vehicle/component/separate

technical unit2

3.1.	Location of that marking:

4.	Category of vehicle:

5.	Name and address of manufacturer:

6.	In the case of components and separate technical units, location and method of 
	affixing of the approval mark:

7.	Address(es) of assembly plant(s):

8.	Additional information (where applicable): See appendix below

9.	Technical Service responsible for carrying out the tests:

10.	Date of test report:

11.	Number of test report:

12.	Remarks (if any): See appendix below

13.	Place:

14.	Date:

15.	Signature:

16.	The index to the information package lodged with the Approval Authority, 
	which may be obtained on request is attached:

17.	Reasons for extension:

Appendix to type approval communication form No.........

concerning the type approval of a vehicle under UN Regulation No. 10

1.	Additional information:

2.	Electrical system rated voltage: ..............................................V. pos/neg ground2

3.	Environment of charging for EV and PHEV:

Type of vehicle valid for residential environment:

Type of vehicle valid for non-residential environment:

34.	Type of bodywork:

45.	List of electronic systems installed in the tested vehicle(s) not limited to the items

in the information document:

45.1.	Vehicle equipped with 24 GHz short-range radar equipment: yes/no/optional2

56.	Laboratory accredited to ISO 17025 and recognized by the Approval Authority 
	responsible for carrying out the tests:

67.	Remarks: (e.g. valid for both left-hand drive and right-hand drive vehicles):		"

Annex 4,

Paragraph 2., amend to read:

"2.		Vehicle state during tests

For two-wheeled vehicles, a non-conductive insulating support with a thickness of 5 – 20mm shall be used between stand and ground plane."

Paragraph 2.1.1., amend to read:

"2.1.1.		Engine

The engine shall be in operation according to CISPR 12.

For vehicle with an electric propulsion motor or hybrid propulsion system, if this is not appropriate (e.g. in case of busses, trucks, two- and three wheel vehicles), transmission shafts, belts or chains may be disconnected to achieve the same operation condition for the propulsion.

If operating mode defined in CISPR 12 (e.g. 40 km/h) does not activate all traction motors or auxiliary battery charging engines, an alternative, steady state operating mode shall be agreed between the Technical Service and vehicle manufacturer. "

Paragraph 2.2., amend to read:

"2.2.		Vehicle in configuration "REESS charging mode coupled to the power grid".

The vehicle shall be tested in the charging mode configuration (if available on vehicle) as defined in flowchart of figure 1

Figure 1

Charging mode configuration for Annex 4

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole frequency range measurement (this may lead to splitting the measurement into different sub-bands with the need to discharge the vehicle's traction battery before starting the next sub-bands).

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal maximum rated charging/input current value for AC charging.

If the current consumption can be adjusted, then the current shall be set to at least 80 20 per cent of its nominal value or to a minimum of 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities.

In case of multiple batteries the average state of charge must be considered.

The vehicle shall be immobilized, the engine(s) (ICE and / or electrical engine) shall be OFF and in charging mode. All other equipment which can be switched ON by the driver or passengers shall be OFF.

The test set-up for the connection of the vehicle in configuration "REESS charging mode coupled to the power grid" is shown in Figures 3a to 3h (depending of AC or DC power charging mode, location of charging plug and charging with or without communication) of Appendix 1 to this Aannex."

Paragraph 2.3.3., amend to read:

"2.3.3.	Power charging harness

The power charging harness shall be placed in a straight line between the AMN(s) and the vehicle charging plug and shall be routed perpendicularly to the vehicle longitudinal axis (see Figure 3da and Figure 3c). The projected harness length from the side of the AMN(s) to the side of the vehicle shall be 0,8 (+0,2 / -0) m as shown in Figure 3db and Figure 3ed.

For a longer harness the extraneous length shall be “Z-folded” in a less than 0,5 m width approximately around the middle of the AMN to vehicle distance. If it is impractical to do so because of harness bulk or stiffness, or because the testing is being done at a user’s installation, the disposition of the excess harness shall be precisely noted in the test report.

For a longer cable, the extraneous length shall be “Z-folded” symmetrically. No contact or overlap is allowed between windings. The width of the Z-folded cable shall be between 500 mm and 1 000 mm. If it is impractical to do so because of cable bulk or stiffness, or because the testing is being done at a user's installation, the disposition of the excess cable length shall be precisely noted in the test report.

The charging harness at the vehicle side shall hang vertically at a distance of 100 (+200 / -0) mm from the vehicle body.

The whole harness shall be placed on a non-conductive, low relative permittivity (dielectric-constant) material (r ≤ 1,4), at (100  25) mm above the ground plane (ALSE) or floor (OTS)."

Paragraphs 2.4.3. and 2.4.4., amend to read:

"2.4.3.	Asymmetric artificial network

Local/private communication lines connected to signal/control ports and lines connected to wired network ports shall be applied to the vehicle through AAN(s).

The various AAN(s) to be used are defined in Appendix 8, clause 5:

-	Clause 5.1 for signal/control port with symmetric lines;

-	Clause 5.2 for wired network port with PLC on power lines;

-	Clause 5.3 for signal/control port with PLC (technology) on control pilot; and

-	Clause 5.4 for signal/control port with control pilot.

The AAN(s) shall be mounted directly on the ground plane. The case of the AAN(s) shall be bonded to the ground plane (ALSE) or connected to the protective earth (OTS, e.g. an earth rod).

The measuring port of each AAN shall be terminated with a 50  load.

If a charging station is used, AAN(s) are not required for the signal/control ports and/or for the wired network ports. The local/private communication lines between the vehicle and the charging station shall be connected to the associated auxiliary equipment on the charging station side to work as designed. If communication is emulated and if the presence of the AAN prevents proper communication then no AAN should be used

2.4.4.	Power charging / local/private communication harness

The power charging local/private communication harness shall be laid out in a straight line between the AMN(s) / DC-charging-AN(s) / AAN(s) and the vehicle charging socket and shall be routed perpendicularly to the vehicle’s longitudinal axis (see Figure 3fe and Figure 3g). The projected harness length from the side of the AMN(s) to the side of the vehicle shall be 0,8 (+0,2 / -0) m as shown in Figure 3f and Figure 3h.

For a longer harness the extraneous length shall be “Z-folded” in less than 0,5 m width. If it is impractical to do so because of harness bulk or stiffness, or because the testing is being done at a user installation, the disposition of the excess harness shall be precisely noted in the test report.

For a longer cable, the extraneous length shall be “Z-folded” symmetrically. No contact or overlap is allowed between windings. The width of the Z-folded cable shall be between 500 mm and 1 000 mm. If it is impractical to do so because of cable bulk or stiffness, or because the testing is being done at a user's installation, the disposition of the excess cable length shall be precisely noted in the test report.

The power charging local/private communication harness at vehicle side shall hang vertically at a distance of 100 (+200 / -0) mm from the vehicle body.

The whole harness shall be placed on a non-conductive, low relative permittivity (dielectric-constant) material (r ≤ 1,4), at (100  25) mm above the ground plane (ALSE) or floor (OTS)."

Paragraphs 4.3. and 4.4., amend to read:

"4.3.	The measurements shall be performed with a spectrum analyser or a scanning receiver. The parameters to be used are defined in Table 1 and Table 2.

Spectrum analysers and FFT-based instruments, that meet the requirements of CISPR 16-1-1, may be used for conformity measurements. FFT-based measuring instruments shall continuously record and evaluate the signal during the measurement time. If using FFT-based instruments, the minimum measurement time shall be 1 s per analysis frequency band (in real-time mode) of the FFT instrument.

# Table 1

Spectrum analyser parameters

| Frequency range MHz | Peak detector |  | Quasi-peak detector |  | Average detector |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RBW at -3 dB | Minimum scan time | RBW at -6 dB | Minimum scan time | RBW at -3 dB | Minimum scan time |
| 30 to 1,000 | 100/120 kHz | 100 ms/MHz | 120 kHz | 20 s/MHz | 100/120 kHz | 100 ms/MHz |

Note: If a spectrum analyser is used for peak measurements, the video bandwidth shall be at least three times the resolution bandwidth (RBW).

# Table 2

Scanning receiver parameters

| Frequency range MHz | Peak detector |  |  | Quasi-peak detector |  |  | Average detector |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW at -6 dB | Step size a | Minimum scan dwell time | BW at -6 dB | Step size a | Minimum Dwell time | BW at -6 dB | Step size a | Minimum scan dwell time |
| 30 to 1,000 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |

a  For purely broadband disturbances, the maximum frequency step size may be increased up to a value not greater than the bandwidth value.

4.4.	Measurements

The Technical Service shall perform the test at the intervals specified in the CISPR 12 standard throughout the frequency range 30 to 1,000 MHz.

Alternatively, if the manufacturer provides measurement data for the whole frequency band from a test laboratory accredited to the applicable parts of ISO 17025 and recognized by the Type Approval Authority, the Technical Service may divide the frequency range in 14 frequency bands 30–34, 34–45, 45–60, 60–80, 80–100, 100–130, 130–170, 170–225, 225–300, 300–400, 400–525, 525–700, 700–850 and 850–1,000 MHz and perform tests at the 14 frequencies giving the highest emission levels within each band to confirm that the vehicle meets the requirements of this Aannex.

For configuration "REESS charging mode coupled to the power grid", if the manufacturer provides measurement data for the whole frequency band from a test laboratory accredited to the applicable parts of ISO 17025 and recognized by the Type Approval Authority for all the available charging modes configurations defined in 2.2, the Technical Service may perform tests only for one of the available charging mode configuration defined in 2.2 and divide the frequency range in 14 frequency bands 30–34, 34–45, 45–60, 60–80, 80–100, 100–130, 130–170, 170–225, 225–300, 300–400, 400–525, 525–700, 700–850 and 850–1,000 MHz and perform tests at the 14 frequencies giving the highest emission levels within each band to confirm that the vehicle meets the requirements of this Annex.

In the event that the limit is exceeded during the test, investigations shall be made to ensure that this is due to the vehicle and not to background radiation."

Paragraph 4.6., amend to read:

"4.6.	Antenna position

Measurements shall be made on the left and right sides of the vehicle.

The horizontal distance is from the reference point of the antenna to the nearest part of the vehicle body.

Multiple antenna positions may be required (both for 10 m and 3 m antenna distance) depending on the vehicle length. The same positions shall be used for both horizontal and vertical polarization measurements. The number of antenna positions and the position of the antenna with respect to the vehicle shall be documented in the test report.

- If the length of the vehicle is smaller than the 3 dB beamwidth of the antenna, only one antenna position is necessary. The antenna shall be aligned with the middle of the total vehicle (see Figure 4);

- If the length of the vehicle is greater than the 3 dB beamwidth of the antenna, multiple antenna positions are necessary in order to cover the total length of the vehicle (see Figure 5). The number of antenna positions shall allow to meet the following condition:

(1)

With:

N: Number of antenna positions;

D: Measurement distance (3 m or 10 m);

2β: 3 dB antenna beamwidth angle in the plane parallel to ground (i.e. the E-plane beamwidth angle when the antenna is used in horizontal polarization, and the H-plane beamwidth angle when the antenna is used in vertical polarization);

L: Total vehicle length; covers the whole dimensions including tires, bumpers and lights, etc.

Depending of the chosen values of N (number of antenna positions) different set-up shall be used:

if N=1 (only one antenna position is necessary) and the antenna shall be aligned with the middle of the total vehicle length (see Figure 4).

if N>1 (more than one antenna position is necessary) and multiple antenna positions are necessary in order to cover the total length of the vehicle (see Figure 5). The antenna positions shall be symmetric in regard to the vehicle perpendicular axis."

Annex 4, Appendix 1, amend to read:

"Annex 4 – Appendix 1

# Figure 2
Position of antenna in relation to the vehicle:

Figure 2a
Dipole antenna in position to measure the vertical radiation components

Figure 2b
Dipole antenna in position to measure the horizontal radiation components

# Figure 3
Vehicle in configuration "REESS charging mode" coupled to the power grid:

Example of test setup for vehicle with socket located on vehicle side 
(charging mode 1 or 2, AC powered, without communication).

# Figure 3a

Figure 3b

Legend:

1:	Vehicle under test.

2:	Insulating support.

3:	Charging harness (including EVSE for charging mode 2).

4:	AMN(s) or DC-charging-AN(s) grounded.

5:	Power mains socket.

# Figure 3a

# Figure 3b

Key

| 1 | vehicle under test |
| --- | --- |
| 2 | insulating support |
| 3 | charging cable (including EVSE for charging mode 2) |
| 4 | artificial mains network(s) grounded |
| 5 | power mains socket (see 7.3.2.2) |
| 6 | extraneous length Z-folded |

NOTE:	The cable between the AC mains and the AMN need not be aligned in same direction as the cable between the AMN and the EV.

Example of test setup for vehicle with socket located front / rear of vehicle (charging mode 1 or 2, AC powered, without communication).

# Figure 3c

# Figure 3d

Legend:

1:	Vehicle under test.

2:	Insulating support.

3:	Charging harness (including EVSE for charging mode 2).

4:	AMN(s) or DC-charging-AN(s) grounded.

5:	Power mains socket.

# Figure 3c

# Figure 3d

Key

| 1 | vehicle under test |
| --- | --- |
| 2 | insulating support |
| 3 | charging cable (including EVSE for charging mode 2) |
| 4 | artificial mains network(s) grounded |
| 5 | power mains socket (see 7.3.2.2) |
| 6 | extraneous length Z-folded |

NOTE:	The cable between the AC mains and the AMN need not be aligned in same direction as the cable between the AMN and the EV.

Example of test setup for vehicle with socket located on vehicle side 
(charging mode 3 or mode 4, with communication)

Figure 3e

Figure 3f

Legend:

1:	Vehicle under test.

2:	Insulating support.

3:	Charging harness with local/private communication lines.

4:	AMN(s) or DC-charging-AN(s) grounded.

5:	Power mains socket.

6:	AAN(s) grounded (optional).

7:	Charging station.

Figure 3e

Figure 3f

Key

| 1 | vehicle under test |
| --- | --- |
| 2 | insulating support |
| 3 | charging harness with communication lines |
| 4 | AMN(s) or DC-charging-AN(s), grounded |
| 5 | power mains / supply socket (optional, see 7.3.3.2) |
| 6 | AAN(s), grounded (optional, not represented in the front view) |
| 7 | charging station (can be emulated) |
| 8 | communication lines |
| 9 | communication module |
| 10 | power cable |
| 11 | extraneous length Z-folded |

NOTE	: The cable between the AC/DC mains/supply and the AMN/DC-charging-AN need not be aligned in same direction as the cable between the AMN/DC-charging-AN and the EV.

Example of test setup for vehicle with socket located front / rear of vehicle (charging mode 3 or mode 4, with communication)

Figure 3g

Figure 3h

Legend:

1:	Vehicle under test.

2:	Insulating support.

3:	Charging harness with local/private communication lines.

4:	AMN(s) or DC-charging-AN(s) grounded.

5:	Power mains socket.

6:	AAN(s) grounded (optional).

7:	Charging station.

Figure 3g

Figure 3h

Key

| 1 | vehicle under test |
| --- | --- |
| 2 | insulating support |
| 3 | charging harness with communication lines |
| 4 | AMN(s) or DC-charging-AN(s), grounded |
| 5 | power mains / supply socket (optional, see 7.3.3.2) |
| 6 | AAN(s), grounded (optional, not represented in the front view) |
| 7 | charging station (can be emulated) |
| 8 | communication lines |
| 9 | communication module |
| 10 | power cable |
| 11 | extraneous length Z-folded |

NOTE:	The cable between the AC/DC mains/supply and the AMN/DC-charging-AN need not be aligned in same direction as the cable between the AMN/DC-charging-AN and the EV.

…"

Annex 5,

Paragraph 1.3., delete.

Paragraph 1.4., renumber and amend to read:

"1.34.	As an alternative fFor vehicles of category L the measurement location can be chosen according to Annex 4, paragraphs 3.1. and 3.2."

Paragraph 2.1., amend to read:

"2.1.	The ignition switch shall be switched on. The engine shall not be operating.

For two-wheeled vehicles, a non-conductive insulating support with a thickness of 5 – 20mm shall be used between stand and ground plane."

Paragraph 4.3., amend to read:

"4.3.		The measurements shall be performed with a spectrum analyser or a scanning receiver. The parameters to be used are defined in Table 1 and Table 2.

Spectrum analysers and FFT-based instruments, that meet the requirements of CISPR 16-1-1, may be used for conformity measurements. FFT-based measuring instruments shall continuously record and evaluate the signal during the measurement time. If using FFT-based instruments, the minimum measurement time shall be 1 s per analysis frequency band (in real-time mode) of the FFT instrument.

# Table 1
Spectrum analyser parameters

| Frequency range MHz | Peak detector |  | Average detector |  |
| --- | --- | --- | --- | --- |
|  | RBW at -3 dB | Minimum scan time | RBW at -3 dB | Minimum scan time |
| 30 to 1,000 | 100/120 kHz | 100 ms/MHz | 100/120 kHz | 100 ms/MHz |

Note: If a spectrum analyser is used for peak measurements, the video bandwidth shall be at least three times the resolution bandwidth (RBW).

# Table 2
Scanning receiver parameters

| Frequency range MHz | Peak detector |  |  | Average detector |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BW at -6 dB | Step size | Minimum scan Dwell time | BW at -6 dB | Step size | Minimum scan Dwell time |
| 30 to 1,000 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 5 ms |

"

Paragraph 4.6., amend to read:

"4.6.	Antenna position

Measurements shall be made on the left and right sides of the vehicle.

The horizontal distance is from the reference point of the antenna to the nearest part of the vehicle body.

Multiple antenna positions may be required (both for 10 m and 3 m antenna distance) depending on the vehicle length. The same positions shall be used for both horizontal and vertical polarization measurements. The number of antenna positions and the position of the antenna with respect to the vehicle shall be documented in the test report.

- 	If the length of the vehicle is smaller than the 3 dB beamwidth of the antenna, only one antenna position is necessary. The antenna shall be aligned with the middle of the total vehicle (see Figure 1).

- 	If the length of the vehicle is greater than the 3 dB beamwidth of the antenna, multiple antenna positions are necessary in order to cover the total length of the vehicle (see Figure 2). The number of antenna positions shall allow to meet the following condition:

(1)

With:

N: number of antenna positions.

D: measurement distance (3 m or 10 m).

2β: 3 dB antenna beamwidth angle in the plane parallel to ground (i.e. the E-plane beamwidth angle when the antenna is used in horizontal polarization, and the H-plane beamwidth angle when the antenna is used in vertical polarization).

L: Total vehicle length covers the whole dimensions including tires, bumpers and lights, etc.

Depending of the chosen values of N (number of antenna positions) different set-up shall be used:

if N=1 (only one antenna position is necessary) and the antenna shall be aligned with the middle of the total vehicle length (see Figure 1).

if N>1 (more than one antenna position is necessary) and multiple antenna positions are necessary in order to cover the total length of the vehicle (see Figure 2). The antenna positions shall be symmetric in regard to the vehicle perpendicular axis."

Annex 6,

Paragraphs 1.1. to 1.3., amend to read:

"1.1.	The test method described in this Aannex shall only be applied to vehicles. This method concerns both configurations of vehicle:

(a)	Other than "REESS charging mode coupled to the power grid";

(b)	"REESS charging mode coupled to the power grid".

A vehicle is considered to be a “large vehicle”, if it is longer than 12 m and/or wider than 2.60 m and/or higher than 4.00 m.

1.2.	Regular Ttest method

This test is intended to demonstrate the immunity of the vehicle electronic systems. The vehicle shall be subject to electromagnetic fields as described in this Aannex. The vehicle shall be monitored during the tests.

If not otherwise stated in this Aannex the test shall be performed according to ISO 11451-2. in an ALSE:

      - with front irradiation for vehicle not considered as “large vehicles”.  Rear irradiation is specified in paragraph 5.1.3.

      - with front irradiation and with additional antenna positions for “large vehicles”. Additional antenna position(s) shall be chosen by the manufacturer in conjunction with the Type Approval Authority after considering the distribution of electronic systems with immunity related functions and the layout of any wiring harness. Tests shall be performed with levels defined in paragraph 6.4.2.1. of this Regulation. For REESS charging mode, only the electronic systems and wiring harnesses required for charging mode shall be considered for antenna positions.

1.3.	Alternative test methods

The test may be alternatively performed in an outdoor test site for all vehicles (including “large vehicles”). The test facility shall comply with (national) legal requirements regarding the emission of electromagnetic fields. The test shall be performed according to ISO 11451-2 in an OTS:

      - with front irradiation for vehicle not considered as “large vehicles”

      - with front irradiation and with additional antenna positions for “large vehicles”. Additional antenna position(s) shall be chosen by the manufacturer in conjunction with the Type Approval Authority after considering the distribution of electronic systems with immunity related functions and the layout of any wiring harness. Tests shall be performed with levels defined in paragraph 6.4.2.1. of this Regulation.

For “large vehicles”, the following alternative methods may be chosen by the manufacturer in conjunction with the Type Approval Authority:

      - Harness excitation methods (BCI) according to ISO 11451-4 in the frequency range 20 to 2,000 MHz and immunity to external sources according to ISO 11451-2 in ALSE or OTS in the frequency range 2,000 to 6,000 MHz with additional antenna position(s). Additional antenna positions shall be chosen by the manufacturer in conjunction with the Type Approval Authority after considering the distribution of electronic systems with immunity related functions and the layout of any wiring harness. Tests shall be performed with levels defined in paragraph 6.4.2.1. of this Regulation.

      - Harness excitation methods (BCI) according to ISO 11451-4 in the frequency range 20 to 2,000 MHz and ESA immunity to external sources according to Annex 9 in the frequency range 2,000 to 6,000 MHz for all ESA involved in immunity related functions. ESA involved in immunity related functions shall be chosen by the manufacturer in conjunction with the Type Approval Authority. Vehicle test shall be performed with levels defined in paragraph 6.4.2.1. of this Regulation. ESA shall be performed with levels defined in paragraph 6.8.2.1. of this Regulation.

      - Immunity to external sources according to ISO 11451-2 in ALSE or OTS in the frequency range 20 to 6,000 MHz with front irradiation and ESA immunity to external sources according to Annex 9 in the frequency range 20 to 6,000 MHz for ESA with immunity related functions out antenna beamwidth. Concerned ESA shall be chosen by the manufacturer in conjunction with the Type Approval Authority Vehicle test shall be performed with levels defined in paragraph 6.4.2.1. of this Regulation. ESA tests shall be performed with levels defined in paragraph 6.8.2.1. of this Regulation.

[Alternative method using ESA immunity to external sources according to Annex 9 does not require E-marking of the concerned ESA. The test report shall be prepared or approved by a laboratory accredited to ISO 17025 and recognized by the Approval Authority responsible for carrying out the tests and provided along with the information document shown in Annex 2B.]

If a vehicle is longer than 12 m and/or wider than 2.60 m and/or higher than 4.00 m, BCI (bulk current injection) method according to ISO 11451-4 shall be used in the frequency range 20 to 2,000 MHz with levels defined in paragraph 6.8.2.1. of this Regulation."

Insert a new paragraph 1.4., to read:

"1.4 	Applicability of test methods:

Figure 1

Type approval procedures for testing the immunity of vehicles to electromagnetic radiation

Alternative

No

Yes

Regular or alternative test method

One of the following test methods shall be used

Vehicle longer than 12 m and/or wider than 2.60 m and/or higher than 4.00 m

Regular

Regular

Alternatives

Alternatives

Vehicle test according to ISO 11451-4 BCI (20MHz – 2 GHz)

+ according to ISO 11451-2 in ALSE or OTS (additional antenna positions for ESA with immunity related functions) (2 – 6 GHz)

Vehicle test according to ISO 11451-2 in ALSE

Vehicle test according to ISO 11451-2 in OTS

Vehicle test according to ISO 11451-4 BCI (20MHz – 2 GHz)

+ Component test according to R10 Annex 9 (2 – 6 GHz)

Vehicle test according to ISO 11451-2 in ALSE or OTS (20MHz – 6 GHz)

+ Component test according to R10 Annex 9 (20 MHz – 6 GHz) for each ESA with immunity related functions outside antenna beamwidth

Vehicle test according to ISO 11451-2 in ALSE (additional antenna positions for ESA with immunity related functions)

Vehicle test according to ISO 11451-2 in OTS (additional antenna positions for ESA with immunity related functions)

"

Paragraph 2., amend to read:

"2.	Vehicle state during tests

For two-wheeled vehicles, a non-conductive insulating support with a thickness of 5 – 20mm shall be used between stand and ground plane."

Paragraph 2.1.1.2., amend to read:

"2.1.1.2.	Basic vehicle conditions

The paragraph defines minimum test conditions (as far as applicable) and failure criteria for vehicle immunity tests. Other vehicle systems, which can affect immunity related functions, shall be tested in a way to be agreed between manufacturer and Technical Service.

| "50 km/h mode" vehicle test conditions | Failure criteria |
| --- | --- |
| Vehicle speed 50 km/h (respectively 25 km/h for L1, L2 vehicles) 20 per cent (vehicle driving the rollers). If the vehicle is equipped with a cruise control system, it shall be used to maintain the required constant vehicle speed and maintained without any deactivation. | Speed variation greater than 10 per cent of the nominal speed. In case of automatic gearbox: change of gear ratio inducing a speed variation greater than 10 per cent of the nominal speed. |
| Dipped beams ON (manual mode) | Lighting OFF (front light and rear light) |
| Specific warning (e.g Rotating/flashing light, signaling bar, siren…) ON | Specific warning OFF |
| Cluster operate in normal mode | Unexpected warning Inconsistent variation of the odometer |
| Rear view system | Unexpected movement of rear view mirror Loss or freezing of the display (CMS) |
| Front wiper ON (manual mode) maximum speed | Complete stop of front wiper |
| Direction indicator on driver's side ON | Frequency change (lower than 0.75 Hz or greater than 2.25 Hz). Duty cycle change (lower than 25 per cent or greater than 75 per cent). |
| Adjustable suspension in normal position | Unexpected significant variation |
| Driver's seat and steering wheel in medium position | Unexpected variation greater than 10 per cent of total range |
| Alarm unset | Unexpected activation of alarm |
| Horn OFF | Unexpected activation of horn |
| Airbag and safety restraint systems operational with inhibited passenger airbag if this function exists | Unexpected activation |
| Automatic doors closed | Unexpected opening |
| Adjustable endurance brake lever in normal position | Unexpected activation |
| Brake pedal not depressed | Unexpected activation of brake and unexpected activation of stop lights |
| ADS shall be operational (1) | ADS does not remain in a failure safe mode or expected failure operational mode |
| (1): ADS are turned on by the driver but some or all ADS functions may revert to a mode where system is monitoring sensors but is not actively ‘driving’ the vehicle due to plausibility issues caused by the EMC laboratory environment. |  |

| "Brake mode" vehicle test conditions | Failure criteria |
| --- | --- |
| Vehicle in a state that allows the braking system to operate normally, parking brake released, vehicle speed 0 km/h. Brake pedal depressed to activate the brake function and the stop lights without any dynamic cycle. | Stop lights inactivated during mode Brake warning light ON with loss of brake function. |
| Day running light (DRL) ON | DRL inactivated during mode |
| ADS shall be operational (1) | ADS does not remain in a failure safe mode or expected failure operational mode |
| (1): ADS are turned on by the driver but some or all ADS functions may revert to a mode where system is monitoring sensors but is not actively ‘driving’ the vehicle due to plausibility issues caused by the EMC laboratory environment. |  |

[

| " Emergency calling systems " vehicle test conditions | Failure criteria |
| --- | --- |
| Accident Emergency Call Systems (AECSs) shall be tested over the air transmission for MSD and voice call via a real PLMN or via a network simulator and using a private safety answering point (PSAP). A manual triggering and operation assessment after and before the test should be performed. During the test, the warning signal device (tell-tale that provides a failure indication) should be monitored. SIM call number should be changed to dedicated PSAP number in order to avoid false calls to the emergency services. | The MSD emission assessment shall include the verification of at least the following: vehicle location data is transmitted correctly, and time stamp is transmitted correctly, and vehicle identification number is transmitted correctly. The hands-free voice communication assessment (subjective test) shall include verification of the following: Voice originating inside the vehicle can be clearly heard by the remote listener with satisfactory intelligibility, and Speech of the remote speaker can be clearly heard in the vehicle with satisfactory intelligibility HMI operation assessment shall include a verification of the emergency call status indication operation: system is processing (accident emergency call is triggered, connection is being set up or data transmission is in progress or completed or voice call is in progress). |

]

| AVAS test conditions | Failure criteria |
| --- | --- |
| Vehicle is tested in an operating state where the AVAS Function/sound active (if applicable) (1) | Loss of AVAS function (sound or system error indicator) |
| (1) This test may be incorporated into Brake or 50 km/h mode if AVAS system is active in these modes. If AVAS system is not operational during these two modes, the operating conditions (e.g. vehicle speed) shall be agreed between the Technical Service and the Vehicle Manufacturer. |  |

"

Insert a new paragraph 2.1.1.5., to read:

"2.1.1.5.	If the vehicle is equipped with an ADS, the immunity test shall demonstrate the vehicle stays in a failure safe mode or expected failure operational mode during the test in 50km/h mode or Brake mode. If it is not possible to activate the ADS due to another function (e.g. turn lamp, wiper, etc), these function(s) may be turned off. Then additional test run(s) may be necessary to cover untested function(s) (e.g. turn lamp, wiper, etc). Automated steering and braking may be deactivated if necessary to ensure a safe test environment."

Paragraph 2.2.1.1., amend to read:

"2.2.1.1.		The vehicle shall be immobilized, the engine(s) (ICE and / or electrical engine) shall be OFF and in charging mode.

The vehicle shall be tested in the charging mode configuration (if available on vehicle) as defined in flowchart of figure 2

Figure 2

Charging mode configuration for Annex 6"

Paragraph 2.2.1.2., amend to read:

"2.2.1.2.	Basic vehicle conditions

The paragraph defines minimum test conditions (as far as applicable) and failures criteria for vehicle immunity tests. Other vehicle systems, which can affect immunity related functions, shall be tested in a way to be agreed between manufacturer and Technical Service.

| "REESS charging mode" vehicle test conditions | Failure criteria |
| --- | --- |
| The REESS shall be in charging mode. The REESS State of charge (SOC) shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole frequency range measurement (this may lead to split the measurement in different sub-bands with the need to discharge the vehicle's traction battery before starting the next sub-bands). If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal maximum rated charging/input current value for AC charging. If the current consumption can be adjusted, then the current shall be set to at least 80 20 per cent of its nominal value or to a minimum of 20 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities. In case of multiple batteries the average state of charge must be considered. | Vehicle sets in motion. Unexpected release of the parking brake. Loss of Parking position for automatic transmission. |

"

Paragraph 2.3.3., amend to read:

"2.3.3.	Power charging harness

The power charging harness shall be placed in a straight line between the AMN(s) and the vehicle charging plug and shall be routed perpendicularly to the vehicle longitudinal axis (see Figure 3d5a and Figure 3e5c). The projected harness length from the side of the AMN(s) to the side of the vehicle shall be 0,8 (+0,2 / -0) m as shown in Figure 3d5b and Figure 3e5d.

For a longer harness the extraneous length shall be “Z-folded” in a less than 0,5 m width approximately around the middle of the AMN to vehicle distance. If it is impractical to do so because of harness bulk or stiffness, or because the testing is being done at a user’s installation, the disposition of the excess harness shall be precisely noted in the test report.

For a longer cable, the extraneous length shall be “Z-folded” symmetrically. No contact or overlap is allowed between windings. The width of the Z-folded cable shall be between 500 mm and 1 000 mm. If it is impractical to do so because of cable bulk or stiffness, or because the testing is being done at a user's installation, the disposition of the excess cable length shall be precisely noted in the test report.

The charging harness at the vehicle side shall hang vertically at a distance of 100 (+200 / -0) mm from the vehicle body.

The whole harness shall be placed on a non-conductive, low relative permittivity (dielectric-constant) material (r ≤ 1,4), at (100  25) mm above the ground plane (ALSE) or floor (OTS)."

Paragraphs 2.4.3. and 2.4.4., amend to read:

"2.4.3.	Asymmetric artificial network

Local/private communication lines connected to signal/control ports and lines connected to wired network ports shall be applied to the vehicle through AAN(s).

The various AAN(s) to be used are defined in Appendix 8, clause 5:

-	Clause 5.1. for signal/control port with symmetric lines;

-	Clause 5.2. for wired network port with PLC on power lines;

-	Clause 5.3. for signal/control port with PLC (technology) on control pilot; and

-	Clause 5.4. for signal/control port with control pilot.

The AAN(s) shall be mounted directly on the ground plane. The case of the AAN(s) shall be bonded to the ground plane (ALSE) or connected to the protective earth (OTS, e.g. an earth rod).

The measuring port of each AAN shall be terminated with a 50  load.

If a charging station is used, AAN(s) are not required for the signal/control ports and/or for the wired network ports. The local/private communication lines between the vehicle and the charging station shall be connected to the associated auxiliary equipment on the charging station side to work as designed. If communication is emulated and if the presence of the AAN prevents proper communication then no AAN should be used

2.4.4.	Power charging / local/private communication harness

The power charging local/private communication harness shall be laid out in a straight line between the AMN(s) / DC-charging-AN(s) / AAN(s) and the vehicle charging socket and shall be routed perpendicularly to the vehicle’s longitudinal axis (see Figure 3f5e and Figure 35g). The projected harness length from the side of the AMN(s) to the side of the vehicle shall be 0,8 
(+0,2 / - 0) m as shown in Figure 5f and Figure 5h.

For a longer harness the extraneous length shall be “Z-folded” in less than 0,5 m width. If it is impractical to do so because of harness bulk or stiffness, or because the testing is being done at a user installation, the disposition of the excess harness shall be precisely noted in the test report.

For a longer cable, the extraneous length shall be “Z-folded” symmetrically. No contact or overlap is allowed between windings. The width of the Z-folded cable shall be between 500 mm and 1 000 mm. If it is impractical to do so because of cable bulk or stiffness, or because the testing is being done at a user's installation, the disposition of the excess cable length shall be precisely noted in the test report.

The power charging local/private communication harness at vehicle side shall hang vertically at a distance of 100 (+200 / -0) mm from the vehicle body.

The whole harness shall be placed on a non-conductive, low relative permittivity (dielectric-constant) material (r ≤ 1,4), at (100  25) mm above the ground plane (ALSE) or floor (OTS)."

Insert a new paragraph 3.4., to read:

"3.4.	If a vehicle is longer than 12 m and/or wider than 2.60 m and/or higher than 4.00 m, and tested according to ISO 11451-2, then additional Reference point(s) shall be chosen by the manufacturer in conjunction with the Type Approval Authority after considering the distribution of electronic systems with immunity related functions and the layout of any wiring harness (see Appendix 1, Figure 5)."

Paragraphs 4.1. and 4.1.1., amend to read:

"4.1.	Frequency range, dwell times, polarization.

The vehicle shall be exposed to electromagnetic radiation in the 20 to 26,000 MHz frequency ranges in vertical polarization.

The test signal modulation shall be:

(a)	AM (amplitude modulation), with 1 kHz modulation and 80 per cent modulation depth in the 20 to 800 MHz frequency range; and

(b)	PM (pulse modulation), Ton 577 µs, period 4,600 µs in the 800 to 2,000 MHz frequency range.

- PM2 (pulse modulation type 2), Ton 3 µs, period 3,333 µs in the 2,700 to 3,100 MHz frequency range; and

(c)	PM3 (pulse modulation type 3), Ton 500 µs, period 1,000 µs in the 800 to 2,700 MHz and the 3,100 to 6,000 MHz frequency ranges.

If not otherwise agreed between Technical Service and vehicle manufacturer.

Frequency step size and dwell time shall be chosen according to
ISO 11451-1.

4.1.1.	The Technical Service shall perform the test at the intervals specified in ISO 11451-1 throughout the frequency range 20 to 26,000 MHz.

Alternatively, if the manufacturer provides measurement to data for the whole frequency band from a test laboratory accredited to the applicable parts of ISO 17025 and recognized by the Type Approval Authority, the Technical Service may choose a reduced number of spot frequencies in the range, e.g. 27, 45, 65, 90, 120, 150, 190, 230, 280, 380, 450, 600, 750, 900, 1,300 and 1,800 MHz to confirm that the vehicle meets the requirements of this Aannex.

If the manufacturer provides measurement data for the whole frequency band from a test laboratory accredited to the applicable parts of ISO 17025 and recognized by the Type Approval Authority for all the charging modes configurations defined in paragraph 2.2.1.1, the Technical Service may perform tests only for one charging mode configurations defined in paragraph 2.2.1.2 and for a reduced number of spot frequencies in the range, e.g. 27, 45, 65, 90, 120, 150, 190, 230, 280, 380, 450, 600, 750, 900, 1,300 and 1,800 MHz to confirm that the vehicle meets the requirements of this Annex.

If a vehicle fails the test defined in this Aannex, it shall be verified as having failed under the relevant test conditions and not as a result of the generation of uncontrolled fields."

Paragraph 5.1.3., amend to read:

"5.1.3.	Test phase

The vehicle shall be positioned with the centre line of the vehicle on the vehicle reference point or line. The vehicle shall normally face a fixed antenna. However, where electronic control units with immunity related functions and the associated wiring harness are predominantly in the rear half of the vehicle, the test should normally be carried out with the vehicle facing away from the antenna and positioned as if it had been horizontally rotated 180° around its centre point, i.e. such that the distance from the antenna to the nearest part of the outer body of the vehicle remains the same. In the case of long“large vehicles” (i.e. excluding vehicles of categories L, M1 and N1), which have electronic control units with immunity related functions and associated wiring harness which are not located in the region illuminated at the default reference point of the regular test method predominantly towards the middle of the vehicle, a then additional reference point(s) may be established. based on either the right side surface or the left side surface of the vehicle. The additional This reference point(s) shall be at the midpoint of the vehicle's length or at one point along the side of the vehicle chosen by the manufacturer in conjunction with the Type Approval Authority after considering the distribution of electronic systems and the layout of any wiring harness.

Such testing may only take place if the physical construction of the chamber permits. The antenna location shall be noted in the test report."

Annex 6, Appendix 1, amend to read:

"Annex 6 – Appendix 1

# Figure 1

# Figure 2

# Figure 3

# [Figure 4

]

# Figure 45

Vehicle in configuration "REESS charging mode coupled to the power grid"

Example of test setup for vehicle with socket located on vehicle side 
(charging mode 1 or 2, AC powered, without communication)

Figure 4a

Figure 4b

Legend:

1	Vehicle under test

2	Insulating support

3	Charging harness (including EVSE for charging mode 2)

4	AMN(s) or DC-charging-AN(s) grounded

5	Power mains socket

Figure 5a

Figure 5b

Key

| 1 | vehicle under test |
| --- | --- |
| 2 | insulating support |
| 3 | charging cable (including EVSE for charging mode 2) |
| 4 | artificial mains network(s) grounded |
| 5 | power mains socket |

NOTE:	The cable between the AC mains and the AMN need not be aligned in the same direction as the cable between the AMN and the EV.

Example of test setup for vehicle with socket located front / rear of vehicle side 
(charging mode 1 or 2, AC powered, without communication)

Figure 4c

Figure 4d

Legend:

1	Vehicle under test

2	Insulating support

3	Charging harness (including EVSE for charging mode 2)

4	AMN(s) or DC-charging-AN(s) grounded

5	Power mains socket

Figure 5c

Figure 5d

Key

| 1 | vehicle under test |
| --- | --- |
| 2 | insulating support |
| 3 | charging cable (including EVSE for charging mode 2) |
| 4 | artificial mains network(s) grounded |
| 5 | power mains socket |

NOTE	The cable between the AC mains and the AMN need not be aligned in the same direction as the cable between the AMN and the EV.

Example of test setup for vehicle with socket located on vehicle side 
(charging mode 3 or mode 4, with communication)

Figure 4e

Figure 4f

Legend:

1	Vehicle under test

2	Insulating support

3	Charging harness with local/private communication lines

4	AMN(s) or DC-charging-AN(s) grounded

5	Power mains socket

6	AAN(s) grounded (optional)

7	Charging station

Figure 5e

Figure 5f

Key

| 1 | vehicle under test |
| --- | --- |
| 2 | insulating support |
| 3 | charging harness with communication lines |
| 4 | AMN(s) or DC-charging-AN(s), grounded |
| 5 | power mains / supply socket (optional) |
| 6 | AAN(s) grounded (optional, not represented in the front view) |
| 7 | charging station (can be emulated) |
| 8 | communication lines |
| 9 | communication module |
| 10 | power cable |

NOTE:	The cable between the AC/DC mains/supply and the AMN/DC-charging-AN need not be aligned in the same direction as the cable between the AMN/DC-charging-AN and the EV.

Example of test setup for vehicle with socket located front / rear of the vehicle side 
(charging mode 3 or mode 4, with communication)

Figure 4g

Figure 4h

Legend:

1	Vehicle under test

2	Insulating support

3	Charging harness with local/private communication lines

4	AMN(s) or DC-charging-AN(s) grounded

5	Power mains socket

6	AAN(s) grounded (optional)

7	Charging station

Figure 5g

Figure 5h

Key

| 1 | vehicle under test |
| --- | --- |
| 2 | insulating support |
| 3 | charging harness with communication lines |
| 4 | AMN(s) or DC-charging-AN(s), grounded |
| 5 | power mains / supply socket (optional, see 7.3.3.2) |
| 6 | AAN(s) grounded (optional, not represented in the front view) |
| 7 | charging station (can be emulated) |
| 8 | communication lines |
| 9 | communication module |
| 10 | power cable |  |

NOTE:	The cable between the AC/DC mains/supply and the AMN/DC-charging-AN need not be aligned in the same direction as the cable between the AMN/DC-charging-AN and the EV."

Annex 7,

Paragraph 2.1., amend to read:

"2.1.	The ESA under test shall be in normal operation mode, preferably in maximum load.

ESAs involved in "REESS charging mode coupled to the power grid" shall be in charging mode.

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole frequency range measurement (this may lead to split the measurement in different sub-bands with the need to discharge the vehicle's traction battery before starting the next sub-bands).

If the test is not performed with a REESS the ESA should be tested at rated current.

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal maximum rated charging/input current value for AC charging.

If the current consumption can be adjusted, then the current shall be set to at least 80 20 per cent of its nominal value or to a minimum of 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities. "

Paragraph 4.3., amend to read:

"4.3.	The measurements shall be performed with a spectrum analyser or a scanning receiver. The parameters to be used are defined in Table 1 and Table 2.

Spectrum analysers and FFT-based instruments, that meet the requirements of CISPR 16-1-1, may be used for conformity measurements. FFT-based measuring instruments shall continuously record and evaluate the signal during the measurement time. If using FFT-based instruments, the minimum measurement time shall be 1 s per analysis frequency band (in real-time mode) of the FFT instrument.

# Table 1
Spectrum analyser parameters

| Frequency range MHz | Peak detector |  | Quasi-peak detector |  | Average detector |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RBW at -3 dB | Minimum scan time | RBW at -6 dB | Minimum scan time | RBW at -3 dB | Minimum scan time |
| 30 to 1,000 | 100/120 kHz | 100 ms/MHz | 120 kHz | 20 s/MHz | 100/120 kHz | 100 ms/MHz |

Note: If a spectrum analyser is used for peak measurements, the video bandwidth shall be at least three times the resolution bandwidth (RBW).

# Table 2
Scanning receiver parameters

| Frequency range MHz | Peak detector |  |  | Quasi-peak detector |  |  | Average detector |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW at -6 dB | Step sizea | Minimum dwell time | BW at -6 dB | Step sizea | Minimum dwell time | BW at -6 dB | Step sizea | Minimum dwell time |
| 30 to 1,000 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 1 s | 120 kHz | 50 kHz | 5 ms |

a  For purely broadband disturbances, the maximum frequency step size may be increased up to a value not greater than the bandwidth value.

Note: For emissions generated by brush commutator motors without an electronic control unit, the maximum step size may be increased up to five times the bandwidth."

Annex 8, paragraph 4.3., amend to read:

"4.3.	The measurements shall be performed with a spectrum analyser or a scanning receiver. The parameters to be used are defined in Tables 1 and 2.

Spectrum analysers and FFT-based instruments, that meet the requirements of CISPR 16-1-1, may be used for conformity measurements. FFT-based measuring instruments shall continuously record and evaluate the signal during the measurement time. If using FFT-based instruments, the minimum measurement time shall be 1 s per analysis frequency band (in real-time mode) of the FFT instrument.

Table 1

Spectrum analyser parameters

| Frequency range MHz | Peak detector |  | Average detector |  |
| --- | --- | --- | --- | --- |
|  | RBW at -3 dB | Minimum scan time | RBW at -3 dB | Minimum scan time |
| 30 to 1,000 | 100/120 kHz | 100 ms/MHz | 100/120 kHz | 100 ms/MHz |

Note: If a spectrum analyser is used for peak measurements, the video band width shall be at least three times the resolution band width (RBW)

# Table 2

Scanning receiver parameters

| Frequency range MHz | Peak detector |  |  | Average detector |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BW at -6 dB | Step size | Minimum dwell time | BW at -6 dB | Step size | Minimum dwell time |
| 30 to 1,000 | 120 kHz | 50 kHz | 5 ms | 120 kHz | 50 kHz | 5 ms |

"

Annex 9, paragraph 1.2.1., amend to read:

"1.2.1.	ESAs may comply with the requirements of any combination of the following test methods at the manufacturer's discretion provided that these results in the full frequency range specified in paragraph 3.1. of this Aannex being covered:

(a)	Absorber chamber test according to ISO 11452-2;

(b)	TEM cell testing according to ISO 11452-3;

(c)	Bulk current injection testing according to ISO 11452-4;

(d)	Stripline testing according to ISO 11452-5;

(e)	800 mm stripline according to paragraph 4.5. of this annex.

(e)	Reverberation chamber test according to ISO 11452-11;

ESAs in configuration "REESS charging mode coupled to the power grid" shall comply with the requirements of the combination of the Absorber chamber test according to ISO 11452-2 and Bulk current injection testing according to ISO 11452-4 at the manufacturer's discretion provided that these results in the full frequency range specified in paragraph 3.1. of this Aannex being covered.

(Frequency range and general test conditions shall be based on ISO 11452-1)."

Paragraph 2.2., amend to read:

"2.2.	The ESA under test shall be switched on and shall be stimulated to be in normal operation condition. It shall be arranged as defined in this Aannex unless individual test methods dictate otherwise.

ESAs involved in "REESS charging mode coupled to the power grid" shall be in charging mode.

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole frequency range measurement (this may lead to split the measurement in different sub-bands with the need to discharge the vehicle's traction battery before starting the next sub-bands).

If the test is not performed with a REESS the ESA should be tested at rated current. If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal value."

Insert a new paragraph 2.3. to read:

"2.3.	The paragraph defines minimum test conditions for ESAs involved in "REESS charging mode coupled to the power grid"

| "REESS charging mode" ESA test conditions | Failure criteria |
| --- | --- |
| The REESS shall be in charging mode. The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole time duration of the measurement (this may lead to the measurement being split into different time slots with the need to discharge the vehicle’s traction battery before starting the next time slot). If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal maximum rated charging/input current value for AC charging. If the current consumption can be adjusted, then the current shall be set to at least 80 20 per cent of its nominal value or to a minimum of 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities. In case of multiple batteries the average state of charge must be considered. | Temporary loss of charging function is allowed, provided that there is no incorrect charging condition (e.g. over-current, overvoltage) and the function can be restored by a simple intervention, without the use of tools, such as turning off/on the DUT, after the disturbance is removed. |

"

Paragraphs 2.3. to 2.5.(former), renumber to read:

"2.34.	Any extraneous equipment required to operate the ESA under test shall not be in place during the calibration phase. No extraneous equipment shall be closer than 1 m from the reference point during calibration.

2.45.	To ensure reproducible measurement results are obtained when tests and measurements are repeated, the test signal generating equipment and its layout shall be to the same specification as that used during each appropriate calibration phase.

2.56.	If the ESA under test consists of more than one unit, the interconnecting cables should ideally be the wiring harnesses as intended for use in the vehicle. If these are not available, the length between the electronic control unit and the AN shall be as defined in the standard. All cables in the wiring harness should be terminated as realistically as possible and preferably with real loads and actuators."

Paragraphs 3.1. and 3.2., amend to read:

"3.1.	Frequency range, dwell times

Measurements shall be made in the 20 to 26,000 MHz frequency range with frequency steps according to ISO 11452-1.

The test signal modulation shall be:

(a)	AM (amplitude modulation), with 1 kHz modulation and 80 per cent modulation depth in the 20 to 800 MHz frequency range; and

(b)	PM (pulse modulation), Ton 577 µs, period 4,600 µs in the 800 to 2,000 MHz frequency range.

- PM2 (pulse modulation type 2), Ton 3 µs, period 3,333 µs in the 2,700 to 3,100 MHz frequency range; and

(c)	PM3 (pulse modulation type 3), Ton 500 µs, period 1,000 µs in the 800 to 2,700 MHz and the 3,100 to 6,000 MHz frequency ranges.

If not otherwise agreed between Technical Service and ESA manufacturer.

Frequency step size and dwell time shall be chosen according to
ISO 11452-1.

3.2.	The Technical Service shall perform the test at the intervals specified in 
ISO 11452-1, throughout the frequency range 20 to 26,000 MHz.

Alternatively, if the manufacturer provides measurement to data for the whole frequency band from a test laboratory accredited to the applicable parts of ISO 17025, and recognized by the Type Approval Authority, the Technical Service may choose a reduced number of spot frequencies in the range, e.g. 27, 45, 65, 90, 120, 150, 190, 230, 280, 380, 450, 600, 750, 900, 1,300, and 1,800 MHz to confirm that the ESA meets the requirements of this Aannex."

Paragraph 4.2., amend to read:

"4.2.	TEM cell testing (see Appendix 21 to this Aannex)"

Paragraph 4.3.2.1., amend to read:

"4.3.2.1.	For ESAs in configuration "REESS charging mode coupled to the power grid", an example of test arrangement (for substitution method) is given in Appendix 43 to this Aannex (figure 1 for substitution method and figure 2 for closed loop method)."

Paragraphs 4.5. to 4.5.2.2., to be deleted.

Insert new paragraphs 4.5. to 4.5.2., to read:

"4.5.	Reverberation chamber test

4.5.1.	Test method

This test method allows the testing of vehicle electrical/electronic systems by exposing an ESA to statistically homogeneous and isotropic electromagnetic fields created by injected and stirred mechanically.

4.5.2.	Test methodology

The test shall be performed according ISO 11452-11.

Unless otherwise specified, the reverberation chamber testing method shall be carried out using a test setup with ground plane."

Annex 9,

Appendix 1, to be deleted.

Appendices 2, 3 and 4, renumber to 1, 2 and 3, accordingly.

Annex 10, paragraph 2., amend to read:

"2.	Immunity against transient disturbances conducted along 12/24 V supply lines.

For configuration other than “REESS charging mode coupled to the power grid”, Aapply the test pulses 1, 2a, 2b, 3a, 3b and 4 according to the International Standard ISO 7637-2 to the supply lines as well as to other connections of ESAs which may be operationally connected to supply lines.

For configuration “REESS charging mode coupled to the power grid”, apply the test pulses 1, 2a, 2b, 3a and 3b according to the International Standard ISO 7637-2 to the supply lines as well as to other connections of ESAs which may be operationally connected to supply lines."

Annex 11,

Paragraph 2.1., amend to read:

"2.1.		The vehicle shall be in configuration "REESS charging mode coupled to the power grid".

For two-wheeled vehicles, a non-conductive insulating support with a thickness of 5 – 20mm shall be used between stand and ground plane.

The vehicle shall be tested in the charging mode configuration (if available on vehicle) as defined in flowchart of figure 1

[

Figure 1

Charging mode configuration for Annex 11]

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole time duration of the measurement (this may lead to the measurement being splitting into different time slots with the need to discharge the vehicle’s traction battery before starting the next time slot). If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal maximum rated charging/input current value for AC charging.

In case of multiple batteries the average state of charge must be considered.

The vehicle shall be immobilized, the engine(s) (ICE and / or electrical engine) shall be OFF and in charging mode.

All other equipment which can be switched ON by the driver or passengers shall be OFF."

Paragraphs 4.2. to 4.5., amend to read:

"4.2.	The limits for single phase or three-phase "REESS charging mode coupled to the power grid" with input current ≤ 16 A per phase are given in Table 34 of paragraph 7.3.2.1. of this Regulation.

4.3.	The limits for single phase or other than balanced three-phase "REESS charging mode coupled to the power grid" with input current > 16 A and ≤ 75 A per phase are given in Table 45 of paragraph 7.3.2.2. of this Regulation.

4.4.	The limits for balanced three-phase "REESS charging mode coupled to the power grid" with input current > 16 A and ≤ 75 A per phase are given in Table 56 of paragraph 7.3.2.2. of this Regulation.

4.5.	For three-phase "REESS charging mode coupled to the power grid" with input current > 16 A and ≤ 75 A per phase, when at least one of the three conditions a), b) or c) described in paragraph 5.2. of IEC 61000-3-12 is fulfilled, then the limits given in Table 67 of paragraph 7.3.2.2. of this Regulation can be applied."

Annex 11,

Appendix 1,

Figure 1b, amend to read:

# "Figure 1b

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	Harmonic analyzer

5 	Power supply"

Figure 1d, amend to read:

# "Figure 1d

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	Harmonic analyzer

5 	Power supply"

Annex 12,

Paragraph 2.1., amend to read:

"2.1.		The vehicle shall be in configuration "REESS charging mode coupled to the power grid".

For two-wheeled vehicles, a non-conductive insulating support with a thickness of 5 – 20mm shall be used between stand and ground plane.

The vehicle shall be tested in the charging mode configuration (if available on vehicle) as defined in flowchart of figure 1

[

Figure 1

Charging mode configuration for Annex 12]

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole time duration of the measurement (this may lead to the measurement being splitting into different time slots with the need to discharge the vehicle’s traction battery before starting the next time slot). If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal maximum rated charging/input current value for AC charging.

In case of multiple batteries the average state of charge must be considered.

The vehicle shall be immobilized, the engine(s) (ICE and / or electrical engine) shall be OFF and in charging mode.

All other equipment which can be switched ON by the driver or passengers shall be OFF."

Annex 12,

Appendix 1,

Figure 1b, amend to read:

# "Figure 1b

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	Flicker analyzer

5 	Impedance simulator

6	Power supply"

# Figure 1d, amend to read:

# "Figure 1d

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	Flicker analyzer

5 	Impedance simulator

6	Power supply"

Annex 13,

Paragraph 2.1., amend to read:

"2.1.		The vehicle shall be in configuration "REESS charging mode coupled to the power grid".

For two-wheeled vehicles, a non-conductive insulating support with a thickness of 5 – 20mm shall be used between stand and ground plane.

The vehicle shall be tested in the charging mode configuration (if available on vehicle) as defined in flowchart of figure 1

[

Figure 1: Charging mode configuration for Annex 13]

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole frequency range measurement (this may lead to splitting the measurement in different sub-bands with the need to discharge the vehicle's traction battery before starting the next sub-bands).

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal maximum rated charging/input current value for AC charging.

If the current consumption can be adjusted, then the current shall be set to at least 80 20 per cent of its nominal value or to a minimum of 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities.

In case of multiple batteries the average state of charge must be considered.

The vehicle shall be immobilized, the engine(s) (ICE and / or electrical engine) shall be OFF and in charging mode.

All other equipment which can be switched ON by the driver or passengers shall be OFF."

Paragraph 3.5., amend to read:

"3.5.		The measurements shall be performed with a spectrum analyser or a scanning receiver. The parameters to be used are defined in Table 1 and Table 2.

Spectrum analysers and FFT-based instruments, that meet the requirements of CISPR 16-1-1, may be used for conformity measurements. FFT-based measuring instruments shall continuously record and evaluate the signal during the measurement time. If using FFT-based instruments, the minimum measurement time shall be 1 s per analysis frequency band (in real-time mode) of the FFT instrument.

# Table 1 
Spectrum analyser parameters

| Frequency range MHz | Peak detector |  | Quasi-peak detector |  | Average detector |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RBW at -3 dB | Minimum scan time | RBW at -6 dB | Minimum scan time | RBW at -3 dB | Minimum scan time |
| 0.15 to 30 | 9/10 kHz | 10 s/MHz | 9 kHz | 200 s/MHz | 9/10 kHz | 10 s/MHz |

Note: If a spectrum analyser is used for peak measurements, the video bandwidth shall be at least three times the resolution bandwidth (RBW)

# Table 2 
Scanning receiver parameters

| Frequency range MHz | Peak detector |  |  | Quasi-peak detector |  |  | Average detector |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW at -6 dB | Step size | Minimum dwell time | BW at -6 dB | Step size | Minimum dwell time | BW at -6 dB | Step size | Minimum dwell time |
| 0.15 to 30 | 9 kHz | 5 kHz | 50 ms | 9 kHz | 5 kHz | 1 s | 9 kHz | 5 kHz | 50 ms |

"

Paragraph 4.2., amend to read:

"4.2.	Measurements shall be performed with average and either quasi-peak or peak detectors. The limits are given in paragraph 7.5. of this Regulation.

Table 78 for AC lines and Table 89 for DC lines. If peak detectors are used a correction factor of 20 dB as defined in CISPR 12 shall be applied."

Insert a new paragraph 4.3., to read:

"4.3.	If the manufacturer provides measurement data for the whole frequency band from a test laboratory accredited to the applicable parts of ISO 17025 and recognized by the Type Approval Authority for all the available charging modes configurations defined in paragraph 2.1, the Technical Service may perform tests only for one of the available charging mode configuration defined in paragraph 2.1 to confirm that the vehicle meets the requirements of this Annex."

Annex 13, Appendix 1,

Figure 1b, amend to read:

# "Figure1b

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	AMN(s) or DC-charging-AN(s) grounded

5 	Power mains socket

6 	Measuring receiver"

Figure 1d, amend to read:

# "Figure 1d

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	AMN(s) or DC-charging-AN(s) grounded

5 	Power mains socket

6 	Measuring receiver"

Annex 14, amend to read:

"Annex 14 	(RESERVED)

Method(s) of testing for emission of radiofrequency conducted disturbances on wired network port from vehicles

1.	General

1.1.	The test method described in this annex shall be applied to vehicles in configuration "REESS charging mode coupled to the power grid".

1.2.	Test method

This test is intended to measure the level of radio frequency conducted disturbances generated by vehicle in configuration "REESS charging mode coupled to the power grid" through its wired network port in order to ensure it is compatible with residential, commercial and light industrial environments.

If not otherwise stated in this annex the test shall be performed according to CISPR 22.

2.	Vehicle/ state during tests

2.1.		The vehicle shall be in configuration "REESS charging mode coupled to the power grid". The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole frequency range measurement (this may lead to splitting the measurement in different sub-bands with the need to discharge the vehicle's traction battery before starting the next sub-bands).

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal value for AC charging.

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal value for DC charging unless another value is agreed with the type approval authorities.

In case of multiple batteries the average state of charge must be considered.

The vehicle shall be immobilized, the engine(s) (ICE and / or electrical engine) shall be OFF and in charging mode.

All other equipment which can be switched ON by the driver or passengers shall be OFF.

3.	Test arrangements

3.1.	The test set-up shall be performed according to paragraph 5. of CISPR 22 for conducted emissions.

3.2.	Measuring location

A shielded enclosure or an absorber lined shielded enclosure (ALSE) or an open area test site (OATS) which complies with the requirements of CISPR 16-1-4 may be used.

3.3.	Local/private communication lines connected to signal/control ports and lines connected to wired network ports shall be applied to the vehicle through AAN(s).

The various AAN(s) to be used are defined in Appendix 8, clause 5:

-	Clause 5.1. for signal/control port with symmetric lines;

-	Clause 5.2. for wired network port with PLC on power lines;

-	Clause 5.3. for signal/control port with PLC (technology) on control pilot; and

-	Clause 5.4. for signal/control port with control pilot.

The AAN(s) shall be mounted directly on the ground plane. The case of the AAN(s) shall be bonded to the ground plane (ALSE) or connected to the protective earth (OTS, e.g. an earth rod).

The measuring port of each AAN shall be terminated with a 50  load.

If a charging station is used, AAN(s) are not required for the signal/control ports and/or for the wired network ports. The local/private communication lines between the vehicle and the charging station shall be connected to the associated equipment on the charging station side to work as designed. If communication is emulated and if the presence of the AAN prevents proper communication then no AAN should be used.

3.4.	The test set-up for the connection of the vehicle in configuration "REESS charging mode coupled to the power grid" is shown in Figures 1a to 1d of Appendix 1 to this annex.

If it is impossible to guarantee the functionality of vehicle, due to introduction of AAN, an alternate method described in CISPR 22 (according to Figures 2a to 2d of Appendix 1 to this annex) shall be applied.

3.5.		The measurements shall be performed with a spectrum analyser or a scanning receiver. The parameters to be used are defined in Table 1 and Table 2.

# Table 1
Spectrum analyser parameters

| Frequency range MHz | Peak detector |  | Quasi-peak detector |  | Average detector |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RBW at -3 dB | Minimum scan time | RBW at -6 dB | Minimum scan time | RBW at -3 dB | Minimum scan time |
| 0.15 to 30 | 9/10 kHz | 10 s/MHz | 9 kHz | 200 s/MHz | 9/10 kHz | 10 s/MHz |

Note: If a spectrum analyser is used for peak measurements, the video bandwidth shall be at least three times the resolution bandwidth (RBW)

# Table 2 
Scanning receiver parameters

| Frequency range MHz | Peak detector |  |  | Quasi-peak detector |  |  | Average detector |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW at -6 dB | Step size | Minimum dwell time | BW at -6 dB | Step size | Minimum dwell time | BW at -6 dB | Step size a | Minimum dwell time |
| 0.15 to 30 | 9 kHz | 5 kHz | 50 ms | 9 kHz | 5 kHz | 1 s | 9 kHz | 5 kHz | 50 ms |

4.	Test requirements

4.1.	The limits apply throughout the frequency range 0.15 to 30 MHz for measurements performed in a shielded enclosure or an absorber lined shielded enclosure (ALSE) or an open area test site (OATS).

4.2.	Measurements shall be performed with average and either quasi-peak or peak detectors. The limits are given in Table 9 of paragraph 7.6. If peak detectors are used a correction factor of 20 dB as defined in CISPR 12 shall be applied. "

Annex 14, Appendix 1, to be deleted.

Annex 15, paragraph 2., amend to read:

"2.	Vehicle state during tests in configuration "REESS in charging mode coupled to the power grid"

The vehicle shall be tested in the charging mode configuration (if available on vehicle) as defined in flowchart of figure 1

Figure 1

Charging mode configuration for Annex 15"

Paragraph 2.1.2., amend to read:

"2.1.2.	Basic vehicle conditions

The paragraph defines minimum test conditions (as far as applicable) and failures criteria for vehicle immunity tests. Other vehicle systems, which can affect immunity related functions, shall be tested in a way to be agreed between manufacturer and Technical Service.

| "REESS charging mode" vehicle test conditions | Failure criteria |
| --- | --- |
| The REESS shall be in charging mode. The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole time duration of the measurement (this may lead to the measurement being split into different time slots with the need to discharge the vehicle’s traction battery before starting the next time slot). If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal maximum rated charging/input current value for AC charging. If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal value or to a minimum of 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities. In case of multiple batteries the average state of charge must be considered. | Vehicle sets in motion. Unexpected release of the parking brake. Loss of Parking position for automatic transmission. |

"

Paragraph 4.2., amend to read:

"4.2.	The vehicle shall be placed directly on the ground plane.

For two-wheeled vehicles, a non-conductive insulating support with a thickness of 5 – 20mm shall be used between stand and ground plane."

Paragraph 5.1.1., amend to read:

"5.1.1.	The test shall be conducted in accordance with method according to IEC 61000-4-4 shall be used to establish the test level requirements. Test shall be performed only at the severity levels given in 7.8.2.1."

Insert a new paragraph 6., to read:

"6.	If the manufacturer provides measurement data for the whole frequency band from a test laboratory accredited to the applicable parts of ISO 17025 and recognized by the Type Approval Authority for all the available charging modes configurations defined in paragraph 2.1, the Technical Service may perform tests only for one of the available charging mode configuration defined in paragraph 2.1 to confirm that the vehicle meets the requirements of this Annex."

# Annex 15, Appendix 1,

# Figure 1b, amend to read:

# "Figure 1b

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	CDN

5 	Fast Transients / Burst generator

6	Power supply"

# Figure 1d, amend to read:

# "Figure 1d

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	CDN

5 	Fast Transients / Burst generator

6	Power supply

"

Annex 16,

Paragraph 2., amend to read:

"2.	Vehicle state during tests in configuration "REESS in charging mode coupled to the power grid"

The vehicle shall be tested in the charging mode configuration (if available on vehicle) as defined in flowchart of figure 1

Figure 1

Charging mode configuration for Annex 16"

Paragraph 2.1.2., amend to read:

"2.1.2.	Basic vehicle conditions

The paragraph defines minimum test conditions (as far as applicable) and failures criteria for vehicle immunity tests. Other vehicle systems, which can affect immunity related functions, shall be tested in a way to be agreed between manufacturer and Technical Service.

| " REESS charging mode" vehicle test conditions | Failure criteria |  |
| --- | --- | --- |
| The REESS shall be in charging mode. The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole time duration of the measurement (this may lead to the measurement being split into different time slots with the need to discharge the vehicle’s traction battery before starting the next time slot).. If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal maximum rated charging/input current value for AC charging. If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal value or to a minimum of 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities. In case of multiple batteries the average state of charge must be considered. | Vehicle sets in motion Unexpected release of the parking brake. Loss of Parking position for automatic transmission. |

"

Paragraph 4.2., amend to read:

"4.2.	The vehicle shall be placed directly on the ground plane.

For two-wheeled vehicles, a non-conductive insulating support with a thickness of 5 – 20mm shall be used between stand and ground plane."

Paragraph 5.1.1., amend to read:

"5.1.1.	The test shall be performed in accordance with method according to IEC 61000-4-5 only at the severity levels specified in 7.9.2.1. shall be used to establish the test level requirements."

Insert a new paragraph 6., to read:

6.	If the manufacturer provides measurement data for the whole frequency band from a test laboratory accredited to the applicable parts of ISO 17025 and recognized by the Type Approval Authority for all the available charging modes configurations defined in 2.1, the Technical Service may perform tests only for one of the available charging mode configuration defined in 2.1 to confirm that the vehicle meets the requirements of this Annex."

# Annex 16, Appendix 1,

# Figure 1b, amend to read:

# "Figure 1b

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	CDN

5 	Surge generator

6 	Power supply"

Figure 1d, amend to read:

"Figure 1d

Legend:

1 	Vehicle under test

2 	Insulating support

3 	Charging harness

4 	CDN

5 	Surge generator

6 	Power supply "

Annex 17,

Paragraph 2.1., amend to read:

"2.1.		The ESA shall be in configuration "REESS charging mode coupled to the power grid".

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole time duration of the measurement (this may lead to the measurement being split into different time slots with the need to discharge the vehicle’s traction battery before starting the next time slot).

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal maximum rated charging/input current value for AC charging."

Paragraphs 4.2. to 4.5., amend to read:

"4.2.	The limits for single phase or three-phase ESAs in configuration "REESS charging mode coupled to the power grid" with input current ≤ 16 A per phase are given in Table 120 of paragraph 7.11.2.1. of this Regulation.

4.3.	The limits for single phase or other than balanced three-phase ESAs in configuration "REESS charging mode coupled to the power grid" with input current > 16 A and ≤ 75 A per phase are given in Table 131 of paragraph 7.11.2.2. of this Regulation.

4.4.	The limits for balanced three-phase ESAs in configuration "REESS charging mode coupled to the power grid" with input current > 16 A and ≤ 75 A per phase are given in paragraph Table 142 of 7.11.2.2. of this Regulation.

4.5.	For three-phase ESAs in configuration "REESS charging mode coupled to the power grid" with input current > 16 A and ≤ 75 A per phase, when at least one of the three conditions a), b) or c) described in paragraph 5.2. of 
IEC 61000-3-12 is fulfilled, then the limits given in Table 153 of paragraph 7.11.2.2. of this Regulation can be applied."

Annex 18, paragraph 2.1., amend to read:

"2.1.		The ESA shall be in configuration "REESS charging mode coupled to the power grid"

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole time duration of the measurement (this may lead to the measurement being split into different time slots with the need to discharge the vehicle’s traction battery before starting the next time slot).

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal maximum rated charging/input current value for AC charging. "

Annex 19,

Paragraph 2.1., amend to read:

"2.1.		The ESA shall be in configuration "REESS charging mode coupled to the power grid".

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole frequency range measurement (this may lead to split the measurement in different sub-bands with the need to discharge the vehicle's traction battery before starting the next sub-bands).

If the test is not performed with a REESS the ESA should be tested at rated current.

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal maximum rated charging/input current value for AC charging.

If the current consumption can be adjusted, then the current shall be set to at least 80 20 per cent of its nominal value or to a minimum of 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities. "

Paragraph 3.4., amend to read:

"3.4.		The measurements shall be performed with a spectrum analyser or a scanning receiver. The parameters to be used are defined in Table 1 and Table 2.

Spectrum analysers and FFT-based instruments, that meet the requirements of CISPR 16-1-1, may be used for conformity measurements. FFT-based measuring instruments shall continuously record and evaluate the signal during the measurement time. If using FFT-based instruments, the minimum measurement time shall be 1 s per analysis frequency band (in real-time mode) of the FFT instrument.

# Table 1

# Spectrum analyser parameters

| Frequency range MHz | Peak detector |  | Quasi-peak detector |  | Average detector |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RBW at -3 dB | Minimum scan time | RBW at -6 dB | Minimum scan time | RBW at -3 dB | Minimum scan time |
| 0.15 to 30 | 9/10 kHz | 10 s/MHz | 9 kHz | 200 s/MHz | 9/10 kHz | 10 s/MHz |

Note: If a spectrum analyser is used for peak measurements, the video bandwidth shall be at least three times the resolution bandwidth (RBW)

# Table 2 
Scanning receiver parameters

| Frequency range MHz | Peak detector |  |  | Quasi-peak detector |  |  | Average detector |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW at -6 dB | Step size | Minimum dwell time | BW at -6 dB | Step size | Minimum dwell time | BW at -6 dB | Step size | Minimum dwell time |
| 0.15 to 30 | 9 kHz | 5 kHz | 50 ms | 9 kHz | 5 kHz | 1 s | 9 kHz | 5 kHz | 50 ms |

"

Paragraph 4.2., amend to read:

"4.2.	Measurements shall be performed with average and either quasi-peak or peak detectors. The limits are given in Table 164 of paragraph 7.13.2.1. of this Regulation for AC lines and in Table 175 of paragraph 7.13.2.2. of this Regulation for DC lines. If peak detectors are used a correction factor of 20 dB as defined in CISPR 12 shall be applied."

Annex 20, amend to read:

"Annex 20 	(RESERVED)

Method(s) of testing for emission of radiofrequency conducted disturbances on wired network port from an ESA

1.	General

1.1.	The test method described in this annex shall be applied to ESAs in configuration "REESS charging mode coupled to the power grid".

1.2.	Test method

This test is intended to measure the level of radio frequency conducted disturbances generated by ESA in configuration "REESS charging mode coupled to the power grid" through its wired network port in order to ensure it is compatible with residential, commercial and light industrial environments.

If not otherwise stated in this annex the test shall be performed according to CISPR 22

2.	ESA state during tests

2.1.		The ESA shall be in configuration "REESS charging mode coupled to the power grid".

The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole frequency range measurement (this may lead to split the measurement in different sub-bands with the need to discharge the vehicle's traction battery before starting the next sub-bands).

If the test is not performed with a REESS the ESA should be tested at rated current.

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal value for AC charging.

If the current consumption can be adjusted, then the current shall be set to at least 80 per cent of its nominal value for DC charging unless another value is  agreed with the type approval authorities.

3.	Test arrangements

3.1	Local/private communication lines connected to signal/control ports and lines connected to wired network ports shall be applied to the vehicle through AAN(s).

The various AAN(s) to be used are defined in Appendix 8, clause 5:

-	Clause 5.1. for signal/control port with symmetric lines;

-	Clause 5.2. for wired network port with PLC on power lines;

-	Clause 5.3. for signal/control port with PLC (technology) on control pilot; and

-	Clause 5.4. for signal/control port with control pilot.

The AAN(s) shall be mounted directly on the ground plane. The case of the AAN(s) shall be bonded to the ground plane (ALSE) or connected to the protective earth (OTS, e.g. an earth rod).

The measuring port of each AAN shall be terminated with a 50  load.

If a charging station is used, AAN(s) are not required for the signal/control ports and/or for the wired network ports. The local/private communication lines between the vehicle and the charging station shall be connected to the associated equipment on the charging station side to work as designed. If communication is emulated and if the presence of the AAN prevents proper communication then no AAN should be used

3.2	Measuring location

A shielded enclosure or an absorber lined shielded enclosure (ALSE) or an open area test site (OATS) which complies with the requirements of CISPR 16-1-4 may be used.

3.3	The test set-up (floor-standing equipment) for the connection of the ESA in configuration "REESS charging mode coupled to the power grid" is shown in Figure 1 of Appendix 1 to this annex.

3.4.		The measurements shall be performed with a spectrum analyser or a scanning receiver. The parameters to be used are defined in Table 1 and Table 2.

# Table 1 
Spectrum analyser parameters

| Frequency range MHz | Peak detector |  | Quasi-peak detector |  | Average detector |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RBW at -3 dB | Minimum scan time | RBW at -6 dB | Minimum scan time | RBW at -3 dB | Minimum scan time |
| 0.15 to 30 | 9/10 kHz | 10 s/MHz | 9 kHz | 200 s/MHz | 9/10 kHz | 10 s/MHz |

Note: 
If a spectrum analyser is used for peak measurements, the video bandwidth shall be at least three times the resolution bandwidth (RBW).

# Table 2 
Scanning receiver parameters

| Frequency range MHz | Peak detector |  |  | Quasi-peak detector |  |  | Average detector |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW at -6 dB | Step size | Minimum dwell time | BW at -6 dB | Step size | Minimum dwell time | BW at -6 dB | Step size | Minimum dwell time |
| 0.15 to 30 | 9 kHz | 5 kHz | 50 ms | 9 kHz | 5 kHz | 1 s | 9 kHz | 5 kHz | 50 ms |

4.	Test requirements

4.1.	The limits apply throughout the frequency range 0.15 to 30 MHz for measurements performed in a shielded enclosure or an absorber lined shielded enclosure (ALSE) or an open area test site (OATS).

4.2.	Measurements shall be performed with average and either quasi-peak or peak detectors. The limits are given in Table 16 of paragraph 7.14.2.1. of this Regulation. If peak detectors are used a correction factor of 20 dB as defined in CISPR 12 shall be applied."

Annex 20, Appendix 1, to be deleted.

Annex 21,

Paragraph 2.1., amend to read:

"2.1.	Basic ESA conditions

The paragraph defines minimum test conditions (as far as applicable) and failures criteria for ESA immunity tests.

| "REESS charging mode" ESA test conditions | Failure criteria |
| --- | --- |
| ESA shall be in configuration "REESS charging mode coupled to the power grid". The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole time duration of the measurement (this may lead to the measurement being split into different time slots with the need to discharge the vehicle’s traction battery before starting the next time slot). If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal maximum rated charging/input current value for AC charging. If the current consumption can be adjusted, then the current shall be set to at least 80 20 per cent of its nominal value or to a minimum of 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities. | Incorrect charging condition (e.g. over-current, overvoltage) Temporary loss of charging function is allowed, provided that there is no incorrect charging condition (e.g. over-current, overvoltage) and the function can be restored by a simple intervention, without the use of tools, such as turning off/on the DUT, after the disturbance is removed. |

"

Paragraph 5.1.1., amend to read:

"5.1.1.	The test shall be conducted in accordance with method according to IEC 61000-4-4 shall be used to establish the test level requirements. Test shall be performed only at the severity levels given in 7.15.2.1."

Annex 22,

Paragraph 2.1., to be deleted.

Paragraph 2.1.2., renumber and amend to read:

"2.1.2.	Basic ESA conditions

The paragraph defines minimum test conditions (as far as applicable) and failures criteria for ESA immunity tests.

| "REESS charging mode" ESA test conditions | Failure criteria |
| --- | --- |
| ESA shall be in configuration "REESS charging mode coupled to the power grid". The state of charge (SOC) of the traction battery shall be kept between 20 per cent and 80 per cent of the maximum SOC during the whole frequency range measurement (this may lead to split the measurement in different sub-bands with the need to discharge the vehicle's traction battery before starting the next sub-bands). If the test is not performed with a REESS the ESA should be tested at rated current. If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal maximum rated charging/input current value for AC charging. If the current consumption can be adjusted, then the current shall be set to at least 20 per cent of its nominal value or to a minimum of 16 A (if the 20 per cent of its nominal value cannot be achieved in the test facility) for DC charging unless another value is agreed with the type approval authorities. | Incorrect charging condition (e.g. over-current, overvoltage) Temporary loss of charging function is allowed, provided that there is no incorrect charging condition (e.g. over-current, overvoltage) and the function can be restored by a simple intervention, without the use of tools, such as turning off/on the DUT, after the disturbance is removed. |

"

Paragraph 5.1.1., amend to read:

"5.1.1.	The test shall be conducted in accordance with method according to IEC 61000-4-4 shall be used to establish the test level requirements. Test shall be performed only at the severity levels given in 7.16.2.1. "

II. 		Justification

- The objective of this new 07 series of amendments to UN Regulation No.10 is to adapt the requirements to the technical progress, including the provisions related to Automated Driving Systems (e.g. ADAS).

- IWG EMC considered updating of references, extending the frequencies range, required test level and other significant aspects, including new immunity related functions (e.g. AVAS, eCall), adding an alternative test method for ‘large and long’ vehicles.

- The proposal aims to reduce the ambiguity in the requirements, to provide consistent interpretation and reduce administrative burden.

- A detailed justification will be provided by means of a separate informal document.

*	In accordance with the programme of work of the Inland Transport Committee for 2023 as outlined in proposed programme budget for 2023 (A/77/6 (Sect. 20), table 20.6), the World Forum will develop, harmonize and update UN Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.

** 		Note by the secretariat: the page numbers refer to the consolidated version of the 06 series of amendments to UN Regulation No. 10 (https://unece.org/fileadmin/DAM/trans/main/wp29/wp29regs/2019/E-ECE-324-Add.9-Rev.6.pdf).

The second number is given merely as an example.

Distinguishing number of the country which has granted/extended/refused or withdrawn approval (see Regulation, provisions on approval).

Strike out what does not apply.